Multi-Hop GRE Tunnel Architecture

- 📑 Section 1: The Strategic Blueprint (Why, What, When)

❓ The WHY: The Core Engineering Problem

Before this lab, connecting two geographically separated offices required expensive, dedicated private circuits (like leased lines or MPLS) where you owned the entire path. To save costs, modern enterprises buy standard public internet packages. However, the public internet presents two non-negotiable architectural roadblocks:

- The RFC 1918 Blockade: ISP routers are hardcoded to drop private address spaces (192.168.x.x, 10.x.x.x) instantly. Your internal corporate data cannot transit the public cloud unassisted.

- The Multicast Barrier: Dynamic routing protocols (like OSPF) form neighbor relationships using multicast packets (224.0.0.5). ISPs drop multicast packets globally to protect their networks.

🧩 The WHAT: Generic Routing Encapsulation (GRE)

GRE is a tunneling protocol that constructs a virtual point-to-point connection over a public IP network. It acts as a logical "cloaking device" by wrapping your private corporate packets and OSPF multicast updates inside a legitimate public IP header.

- The Underlay (Physical Reality): The actual, physical internet path consisting of your edge routers plugged into a sequence of third-party ISP routers.

- The Overlay (Logical Illusion): A virtual direct wire linking your branch and main offices. To your internal network devices and OSPF routing engine, the intermediate ISP cloud completely disappears.

⏱️ The WHEN: Enterprise Use Cases

Deploy GRE tunnels when you need to:

- Connect branch offices securely across cheap, third-party public internet connections.

- Run dynamic routing protocols (OSPF, EIGRP, BGP) across a WAN infrastructure that does not natively support multicast.

- Transport non-routable protocols or specialized network traffic across an intermediate IP network.

- 🗺️ Section 2: The Topology & Subnet Allocation Matrix

Plaintext

[Private Branch LAN]                                                                     [Private Main LAN]

192.168.10.0/24                                                                          192.168.20.0/24

|                                                                                        |

(G0/1)                                                                                   (G0/1)

[ R1 ] (G0/0) <--- 100.1.1.0/30 ---> (G0/0) [ ISP-1 ] (G0/1)

(Branch)                                             |

100.1.2.0/30

|

(G0/0) [ ISP-2 ] (G0/1)

|

100.1.3.0/30

|

[ R2 ] (G0/0) <--- 100.2.2.0/30 ---> (G0/1) [ ISP-3 ] (G0/0)

(Main)

=========================================================================================================

||                               Logical Overlay: GRE Tunnel 0 (10.0.0.0/30)                           ||

=========================================================================================================

- 🛠️ Section 3: Step-by-Step Command Playbook & Explanations

Phase 1: Building the ISP Underlay Cloud (Public Transit)

These configurations simulate the public internet infrastructure. The ISP routers execute their own isolated OSPF routing engine (OSPF 100) to guarantee that the public endpoints (100.1.1.1 and 100.2.2.1) can find each other across the multi-hop core.

🌐 ISP-1 Configuration

Plaintext

ISP-1(config)# interface GigabitEthernet 0/0

ISP-1(config-if)# ip address 100.1.1.2 255.255.255.252

ISP-1(config-if)# no shutdown

- What it does: Provisions the public-facing gateway port connecting directly to the R1 Branch router. no shutdown wakes up the physical layer interface.

Plaintext

ISP-1(config)# interface GigabitEthernet 0/1

ISP-1(config-if)# ip address 100.1.2.1 255.255.255.252

ISP-1(config-if)# no shutdown

- What it does: Sets up the public transit link pointing inward toward the central ISP backbone router (ISP-2).

Plaintext

ISP-1(config)# router ospf 100

ISP-1(config-router)# log-adjacency-changes

ISP-1(config-router)# network 100.1.1.0 0.0.0.3 area 0

ISP-1(config-router)# network 100.1.2.0 0.0.0.3 area 0

- What it does: Starts the ISP's core routing engine. The network statements declare which public transit lines are advertised into the internet core using precise /30 wildcard masks (0.0.0.3).

🌐 ISP-2 Configuration (Central Backbone)

Plaintext

ISP-2(config)# interface GigabitEthernet 0/0

ISP-2(config-if)# ip address 100.1.2.2 255.255.255.252

ISP-2(config-if)# no shutdown

ISP-2(config)# interface GigabitEthernet 0/1

ISP-2(config-if)# ip address 100.1.3.1 255.255.255.252

ISP-2(config-if)# no shutdown

ISP-2(config)# router ospf 100

ISP-2(config-router)# log-adjacency-changes

ISP-2(config-router)# network 100.1.2.0 0.0.0.3 area 0

ISP-2(config-router)# network 100.1.3.0 0.0.0.3 area 0

- What it does: Operates as the middle-man internet transit hop. It bridges the route tracking between the ISP-1 segment and the ISP-3 segment.

🌐 ISP-3 Configuration

Plaintext

ISP-3(config)# interface GigabitEthernet 0/0

ISP-3(config-if)# ip address 100.1.3.2 255.255.255.252

ISP-3(config-if)# no shutdown

ISP-3(config)# interface GigabitEthernet 0/1

ISP-3(config-if)# ip address 100.2.2.2 255.255.255.252

ISP-3(config-if)# no shutdown

ISP-3(config)# router ospf 100

ISP-3(config-router)# log-adjacency-changes

ISP-3(config-router)# network 100.1.3.0 0.0.0.3 area 0

ISP-3(config-router)# network 100.2.2.0 0.0.0.3 area 0

- What it does: Provisions the public interface reaching down into the R2 Main Corporate headquarters network.

Phase 2: Corporate Edge Setup & Internet Access

🏢 R1 (Branch Edge Router) Configuration

Plaintext

R1(config)# interface GigabitEthernet 0/1

R1(config-if)# ip address 192.168.10.1 255.255.255.0

R1(config-if)# no shutdown

- What it does: Configures the internal default gateway for all physical computers sitting inside the local Branch LAN office.

Plaintext

R1(config)# interface GigabitEthernet 0/0

R1(config-if)# ip address 100.1.1.1 255.255.255.252

R1(config-if)# no shutdown

- What it does: Assigns the public-facing internet address provided by the ISP to the external WAN interface port.

Plaintext

R1(config)# ip route 0.0.0.0 0.0.0.0 100.1.1.2

- What it does: Configures the Gateway of Last Resort (Static Default Route). This tells R1: "If an outbound packet isn't destined for our local LAN, forward it directly to the ISP gateway at 100.1.1.2."

🏢 R2 (Main Edge Router) Configuration

Plaintext

R2(config)# interface GigabitEthernet 0/1

R2(config-if)# ip address 192.168.20.1 255.255.255.0

R2(config-if)# no shutdown

R2(config)# interface GigabitEthernet 0/0

R2(config-if)# ip address 100.2.2.1 255.255.255.252

R2(config-if)# no shutdown

R2(config)# ip route 0.0.0.0 0.0.0.0 100.2.2.2

- What it does: Mirrors the setup on the Main office edge. The default route points directly to its own immediate next-hop neighbor, ISP-3 (100.2.2.2).

🛑 CRITICAL UNDERLAY MILESTONE: At this point, you must run ping 100.2.2.1 from R1. The public endpoint ping must succeed across the 3 ISP routers before configuring the GRE parameters.

Phase 3: Constructing the GRE Overlay Tunnels

🏢 R1 (Branch Edge) — GRE Configuration

Plaintext

R1(config)# interface Tunnel 0

- What it does: Spawns a virtual, logical software interface inside the router's RAM. It behaves like a physical interface but has no physical port on the back of the router chassis.

Plaintext

R1(config-if)# ip address 10.0.0.1 255.255.255.252

- What it does: Assigns an internal, private IP address to our virtual wire. This link will be used to establish our enterprise routing neighbor relationships.

Plaintext

R1(config-if)# tunnel source GigabitEthernet 0/0

- What it does: Instructs the router's encapsulation engine which physical interface or local public IP address to stamp as the Source on the outer public delivery header.

Plaintext

R1(config-if)# tunnel destination 100.2.2.1

- What it does: Sets the hard target destination for the outer delivery header. This must point to the public WAN IP of the remote peer (R2).

Plaintext

R1(config-if)# tunnel mode gre ip

- What it does: Explicitly sets the tunneling mechanism to Generic Routing Encapsulation over an IPv4 carrier packet. (Note: This is the default behavior on Cisco IOS, but explicitly committing it prevents config drifts).

🏢 R2 (Main Edge) — GRE Configuration

Plaintext

R2(config)# interface Tunnel 0

R2(config-if)# ip address 10.0.0.2 255.255.255.252

R2(config-if)# tunnel source GigabitEthernet 0/0

R2(config-if)# tunnel destination 100.1.1.1

R2(config-if)# tunnel mode gre ip

- What it does: Builds the reciprocal overlay tunnel terminal endpoint at the main headquarters location, targeting R1's public external interface (100.1.1.1).

Phase 4: Deploying Enterprise Corporate OSPF Over the Overlay

Now that the virtual private link is active, we configure the corporate enterprise routing engine (OSPF 1). This allows the internal office subnets to exchange route paths automatically while bypassing the public ISP networks completely.

🏢 R1 (Branch Edge) — OSPF Setup

Plaintext

R1(config)# router ospf 1

R1(config-router)# router-id 1.1.1.1

- What it does: Instantiates corporate OSPF process 1 and assigns a static 32-bit identity string (1.1.1.1) to simplify troubleshooting.

Plaintext

R1(config-router)# network 10.0.0.0 0.0.0.3 area 0

- What it does: Activates OSPF process 1 directly on the Tunnel 0 interface. OSPF can now successfully transmit its 224.0.0.5 multicast hello packets through the GRE tunnel encapsulation wrapper.

Plaintext

R1(config-router)# network 192.168.10.0 0.0.0.255 area 0

- What it does: Scans the router for interfaces matching the 192.168.10.0 subnet, enables OSPF on the internal LAN interface, and advertises this corporate space across the tunnel to the Main office.

🏢 R2 (Main Edge) — OSPF Setup

Plaintext

R2(config)# router ospf 1

R2(config-router)# router-id 2.2.2.2

R2(config-router)# passive-interface GigabitEthernet 0/1

- What it does: Essential security command. It blocks OSPF multicast traffic from flooding out into the user LAN port where local PCs sit, reducing CPU utilization and mitigating security risks.

Plaintext

R2(config-router)# network 10.0.0.0 0.0.0.3 area 0

R2(config-router)# network 192.168.20.0 0.0.0.255 area 0

- What it does: Pulls the virtual tunnel link and the Main office LAN segment into the corporate backbone area.

- 🔬 Section 4: The Diagnostic & Verification Playbook

Use these three core verification tools to audit your live environment and diagnose structural faults:

1. Auditing the Virtual Interface Status

Plaintext

R1# show interfaces tunnel 0

- What to watch for: The first line must explicitly state: Tunnel0 is up, line protocol is up.

- Diagnostic logic: If status is up but line protocol is down, it means the local router cannot find a valid path in its routing table to reach the configured tunnel destination IP address. Check your base default routes.

2. Confirming the Adjacency State

Plaintext

R1# show ip ospf neighbor

- Expected Output:

Plaintext

Neighbor ID     Pri   State           Dead Time   Address         Interface

2.2.2.2           0   FULL/ -         00:00:35    10.0.0.2        Tunnel0

- Diagnostic logic: The state must register as FULL. Notice the segment designation reads FULL/ -. Because GRE interfaces mimic point-to-point connections, OSPF automatically optimizes behavior by skipping Designated Router (DR) and Backup Designated Router (BDR) segment elections.

3. Reviewing the Enterprise Routing Table Layout

Plaintext

R1# show ip route ospf

- Expected Output:

Plaintext

O    192.168.20.0/24 [110/1001] via 10.0.0.2, 00:04:12, Tunnel0

- Diagnostic logic: This confirms the corporate OSPF deployment succeeded. The remote private network 192.168.20.0/24 is now dynamically registered, explicitly targeted to pass through the virtual Tunnel0 interface wrapper.