To remember

Last octet of subnet mask for each prefix 		Total hosts

/24 = 0						256

/25= 128						128

/26= 192						64

/27= 224						32

/28= 240						16

/29= 248						8

/30 = 252						4

If we have ip address and subnet mask when we need to find that subnets range:

Eg 192.168.1.60 /27

1st that prefix’s hosts number multiple

For /27 hosts= 32

So make 32’s multiple , 0, 32, 64, 96, 128, 160, 192, 224

From the multiple’s , compare with ip’s last value 60, which should be lesser than in multiple table that is 32 here,

So it lies between 32-63. (32’s multiple helps us know the network and broadcast of /27 subnet, the ranges of /27 for 60 to lie in is 32 to 64-1.)

Block size = total hosts

Block size → find value just below IP → network address Network + block size - 1 → broadcast Last usable = broadcast – 1

To check if 2 ip’s are in same subnet

Find network address of both IPs

If same → same subnet

If different → different subnet

Number of subnets from /24

256 ÷ block size = number of subnets

/28 → 256÷16 = 16 subnets

/29 → 256÷8  = 32 subnets

/30 → 256÷4  = 64 subnets

Why /30 exists

Router to router links need exactly 2 addresses

/30 gives exactly 2 usable hosts — no waste

Dept A 60 hosts → /26 block=64 172.16.0.0/26 mask=255.255.255.192 network=.0 broadcast=.63 usable=.1-.62 Dept B 30 hosts → /27 block=32 172.16.0.64/27 mask=255.255.255.224 network=.64 broadcast=.95 usable=.65-.94 Dept C 10 hosts → /28 block=16 172.16.0.96/28 mask=255.255.255.240 network=.96 broadcast=.111 usable=.97-.110 Dept D 2 hosts → /30 block=4 172.16.0.112/30 mask=255.255.255.252 network=.112 broadcast=.115 usable=.113-.114

.

Evidence: