- DNS, HTTP Services, and Multi-Subnet Integration

This document compiles the complete technical overview, architectural rationales, step-by-step configurations, and troubleshooting insights from today's lab session.

- 1. The "Whys" (Core Technical Concepts)

- DNS vs. HTTP Roles: DNS (Domain Name System) operates at Layer 7 as a global phonebook mapping domain names to IP addresses. HTTP/HTTPS also operates at Layer 7 but handles the delivery of web files (like .html files) over TCP ports 80/443.

- Why DNS Record Composition Matters: DNS servers map hostnames to IP addresses only. They have no awareness of specific files or directories. A URL like mywebsite.com/helloworld.html requires DNS to resolve mywebsite.com to an IP first; the client then requests /helloworld.html directly from the web server.

- Why Routing (OSPF) is Essential for Service Traffic: Unlike local DHCP discoveries which use broadcasts, DNS queries, HTTP traffic, and relayed DHCP requests are unicast packets. If routers do not share paths via a routing protocol like OSPF, these packets cannot cross subnets and will drop.

- The Return Path Necessity: A device can successfully receive a packet from a different network, but if its Default Gateway is missing or wrong, it cannot send a reply back. Traffic fails bi-directionally without a valid return path to the local router interface.

- 2. Complete Configuration Guide (The "Hows" & Commands)

Step 1: Physical Server End Devices

You must use dedicated Server objects from the End Devices menu to access server-side daemon profiles.

- DNS Server (Network 20 Side):

  - IP/Subnet: 192.168.20.254 / 255.255.255.0

  - Default Gateway: 192.168.20.1

  - Service Action: Services tab → DNS → Toggle ON. Add an A Record with Name: mywebsite.com and Address: 192.168.10.5.

- Web Server (Network 10 Side):

  - IP/Subnet: 192.168.10.5 / 255.255.255.0

  - Default Gateway: 192.168.10.1

  - Service Action: Services tab → HTTP → Toggle HTTP/HTTPS ON. Click New File, name it helloworld.html, add your text, and click Save.

Step 2: Router Interface Foundations

Plaintext

! ON ROUTER 0 (DHCP Server & Network 10 Gateway)

Router0(config)# interface gig 0/0

Router0(config-if)# ip address 192.168.10.1 255.255.255.0

Router0(config-if)# no shutdown

Router0(config-if)# interface gig 0/1

Router0(config-if)# ip address 192.168.90.1 255.255.255.252

Router0(config-if)# no shutdown

! ON ROUTER 1 (DHCP Relay & Network 20 Gateway)

Router1(config)# interface gig 0/0

Router1(config-if)# ip address 192.168.90.2 255.255.255.252

Router1(config-if)# no shutdown

Router1(config-if)# interface gig 0/1

Router1(config-if)# ip address 192.168.20.1 255.255.255.0

Router1(config-if)# no shutdown

Step 3: Global OSPF Routing

Plaintext

! ON ROUTER 0

Router0(config)# router ospf 1

Router0(config-router)# network 192.168.10.0 0.0.0.255 area 0

Router0(config-router)# network 192.168.90.0 0.0.0.3 area 0

! ON ROUTER 1

Router1(config)# router ospf 1

Router1(config-router)# network 192.168.20.0 0.0.0.255 area 0

Router1(config-router)# network 192.168.90.0 0.0.0.3 area 0

Step 4: DHCP Server Pools & Relay Configuration

Plaintext

! ON ROUTER 0 (Configuring Pools for Both Sides)

Router0(config)# ip dhcp pool LOCAL_NETWORK_10

Router0(dhcp-config)# network 192.168.10.0 255.255.255.0

Router0(dhcp-config)# default-router 192.168.10.1

Router0(dhcp-config)# dns-server 192.168.20.254

Router0(dhcp-config)# exit

Router0(config)# ip dhcp pool REMOTE_NETWORK_20

Router0(dhcp-config)# network 192.168.20.0 255.255.255.0

Router0(dhcp-config)# default-router 192.168.20.1

Router0(dhcp-config)# dns-server 192.168.20.254

Router0(config)# ip dhcp excluded-address 192.168.10.1

Router0(config)# ip dhcp excluded-address 192.168.20.1

! ON ROUTER 1 (Activating the Relay Agent)

Router1(config)# interface gig 0/1

Router1(config-if)# ip helper-address 192.168.90.1

Step 5: Router Infrastructure DNS Resolution (Optional)

Allows the routers themselves to resolve names via the DNS server.

Plaintext

Router0(config)# ip domain-lookup

Router0(config)# ip name-server 192.168.20.254

Router1(config)# ip domain-lookup

Router1(config-router)# ip name-server 192.168.20.254

- 3. Today's Pitfalls & Troubleshooting Ledger

🛑 Mistake 1: Looking for "Services" on Client PCs

- The Symptom: Missing "Services" tab in the configuration dashboard.

- The Fix: Standard endpoint nodes (PCs, Laptops) lack server capabilities. Drag out a generic Server device instead.

🛑 Mistake 2: Missing Local LAN DHCP Pool

- The Symptom: PC0 on Router 0's local network failed to receive an IP address via DHCP.

- The Fix: A DHCP server needs a distinct pool for every subnet it serves. A local pool (LOCAL_NETWORK_10) had to be explicitly declared alongside the remote pool.

🛑 Mistake 3: File Extensions in DNS Name Strings

- The Symptom: Adding helloworld.html into the DNS server's resource records.

- The Fix: Cleaned the entry. DNS names must strictly be domain roots or hostnames (e.g., mywebsite.com). Web paths belong solely in the URL address bar of the web browser client.

🛑 Mistake 4: Ghost/Empty Server IP Allocations

- The Symptom: DNS resolved correctly via nslookup, but all subsequent pings and HTTP connections timed out.

- The Fix: The target IP address 192.168.10.5 had been mapped in the DNS application, but no actual physical Web Server device had been deployed to hold that address. Dragged in a second server and manually bound it to that IP.

🛑 Mistake 5: Gateway Subnet Mismatches

- The Symptom: Continuous Layer 3 ping drops between hosts on the same network or across transit nodes.

- The Fix: Corrected Default Gateways based on local subnets. Devices matching 192.168.10.X must point to .10.1. Devices matching 192.168.20.X must point to .20.1.

- 4. Verification Methods

- Layer 3 Identity Check: Run ipconfig /all on the client PC. Verify it has a valid IP address (not a 169.254.x.x APIPA address) and that the DNS Server field displays 192.168.20.254.

- Isolated Name Resolution Test: Run nslookup mywebsite.com in the PC Command Prompt. If it returns Address: 192.168.10.5, Layer 7 DNS lookup is functioning correctly.

- End-to-End Connectivity Test: Run ping mywebsite.com. If successful, it confirms the client can reach the DNS server, translate the name, and execute an ICMP echo loop with the web server across the OSPF backbone.

- Application Layer Validation: Open the PC's Web Browser application and type mywebsite.com/helloworld.html. The page should display your custom HTML content instantly.