- Day 48 — STP Deep Dive & Toolkit

- 📌 Section 1: Core Mechanics (WHAT, WHY, WHEN, HOW)

🔹 What is PVST+ & Advanced STP Features?

- PVST+ (Per-VLAN Spanning Tree Plus): Cisco’s default Spanning Tree Protocol implementation running an independent 802.1D STP instance for every single VLAN configured on a switch.

- WHY: Legacy 802.1D CST (Common Spanning Tree) maintains only one topology for the entire network, blocking backup trunk links for all traffic. PVST+ allows L2 traffic load-balancing by designating Switch 1 as the Root Bridge for VLAN 10 and Switch 2 as the Root Bridge for VLAN 20.

- WHEN: Deployed in every enterprise L2 network with redundant switch paths to maximize available bandwidth and enforce active topology protection mechanisms.

- HOW: Achieved through priority tuning, edge port acceleration (PortFast), and protection shields (BPDU Guard, BPDU Filter, Root Guard, Loop Guard).

- 🏢 Section 2: Real-World Scenario — Why We Need This

In an enterprise office with two Distribution/Core Switches (SW1 and SW2) and an Access Switch (SW3) connecting user endpoints:

- Without PVST+ Tuning: One trunk link gets blocked for all VLANs. Half of your expensive redundant link capacity sits completely idle.

- With PVST+ Tuning:

  - VLAN 10 (Sales Data): Flows actively through SW1 (Primary Root).

  - VLAN 20 (Voice/Management): Flows actively through SW2 (Primary Root).

  - Both trunk links actively forward traffic simultaneously, doubling network capacity!

- Without STP Security: An employee brings a cheap unmanaged switch from home, plugs both ends into wall sockets, or connects an unauthorized switch with a lower Bridge ID.

  - Result: A catastrophic L2 loop crashes the network, or the rogue device steals the Root Bridge role.

  - Solution: PortFast + BPDU Guard + Root Guard completely secure the L2 environment.

- 🔑 Section 3: Feature Deep Dives & Distinctions

1. PVST+ Priority Calculations & Tuning

STP selects the Root Bridge based on the lowest Bridge ID (BID):

Bridge ID=Bridge Priority+System ID Extension (VLAN ID)+MAC Address

- Default Priority: 32768 (Total BID for VLAN 10 = 32768+10=32778).

- Priority Increments: Must be configured in exact steps of 4096 (0, 4096, 8192, 12288, 16384, 20480, 24576, 28672, 32768).

Commands to Tune Root Bridge:

- spanning-tree vlan X root primary → Automatically sets priority to 24576 (or 4096 lower than current root).

- spanning-tree vlan X root secondary → Automatically sets priority to 28672.

- spanning-tree vlan X priority <0-61440> → Manually sets an exact priority value.

2. Spanning Tree PortFast (Global vs Interface)

PortFast bypasses the standard 30-second STP convergence delay (15s Listening + 15s Learning) and immediately transitions an access port into the Forwarding state upon link up.

- spanning-tree portfast default (Global Config): Enables PortFast on ALL non-trunking access ports across the entire switch. Trunk ports remain unaffected.

- spanning-tree portfast (Interface Config): Explicitly enables PortFast on a specific individual interface.

- ⚠️ Rule: Never enable PortFast on ports connected to other switches, routers, or hubs!

3. BPDU Guard vs. BPDU Filter

Both features protect PortFast ports, but they react completely differently when an unexpected BPDU arrives.

| Feature | Action Upon Receiving BPDU | Best Use Case | Risk Level |
| --- | --- | --- | --- |
| BPDU Guard | Immediately disables the port and puts it into err-disable state. | Standard Security on ALL Access Ports. (Prevents rogue switches). | Zero Risk: Safely stops L2 loops instantly. |
| BPDU Filter (Global) | Disables PortFast on that port; converts it back into a normal STP port (sends/receives BPDUs). | Specialty edge ports where you don't want to shut down the link if a host tests STP. | Low: Falls back to standard STP logic. |
| BPDU Filter (Interface) | Completely silences STP. Stops sending AND ignores all incoming BPDUs entirely. | Specialized ISP/Lab interconnects only. | ⚠️ EXTREME RISK: Completely disables STP safety, causing instant loops if misconfigured! |

💡 Golden Rule for CCNA: Combine spanning-tree portfast default with spanning-tree portfast bpduguard default globally on all access switches!

4. Root Guard vs. Loop Guard

These features protect the infrastructure topology from unexpected bridge role changes and unidirectional link failures.

| Feature | Where to Apply | What it Protects Against | Port State When Triggered |
| --- | --- | --- | --- |
| Root Guard | Designated Ports facing external / customer / downstream switches. | Rogue switch advertising a superior BPDU (lower priority) to steal the Root role. | root-inconsistent (Blocks traffic until superior BPDUs stop). |
| Loop Guard | Non-Designated Ports (Root Ports or Alternate/Blocking Ports). | Unidirectional link failure (e.g., fiber RX wire breaks, so BPDUs stop arriving). | loop-inconsistent (Blocks traffic instead of transitioning port to forwarding). |

- 🗺️ Section 4: Lab Topology Architecture

Plaintext

+-----------------------+

|          SW1          |

|  (Primary Root V10)   |

+-----------+-----------+

/ \

Fa0/1 /   \ Fa0/2

/     \

Fa0/1 /       \ Fa0/1

+-----------------------+         +-----------------------+

|          SW2          |         |          SW3          |

|  (Primary Root V20)   +---------+    (Access Switch)    |

+-----------------------+ Fa0/2   +---+---------------+---+

Fa0/2      |               |

Fa0/3 |               | Fa0/4

|               |

+----+---+      +----+---+

|  PC-1  |      |  PC-2  |

| (VLAN10)      | (VLAN20)

+--------+      +--------+

Exact Port-to-Port Interconnect Matrix

| Link Connection | Local Device & Port | Remote Device & Port | Link Type / Mode |
| --- | --- | --- | --- |
| SW1 ↔ SW2 | SW1 Fa0/1 | SW2 Fa0/1 | 802.1Q Trunk (VLAN 10, 20) |
| SW1 ↔ SW3 | SW1 Fa0/2 | SW3 Fa0/1 | 802.1Q Trunk (VLAN 10, 20) |
| SW2 ↔ SW3 | SW2 Fa0/2 | SW3 Fa0/2 | 802.1Q Trunk (VLAN 10, 20) |
| SW3 ↔ PC-1 | SW3 Fa0/3 | PC-1 NIC | Access Port (VLAN 10) |
| SW3 ↔ PC-2 | SW3 Fa0/4 | PC-2 NIC | Access Port (VLAN 20) |

- 📜 Section 5: Complete CLI Configuration Scripts

🏢 1. Switch 1 (SW1 — Primary Root for VLAN 10)

Plaintext

enable

configure terminal

hostname SW1

! --- Step 1: Create VLANs ---

vlan 10

name Sales_Data

exit

vlan 20

name Voice_Mgmt

exit

! --- Step 2: Configure PVST+ Primary & Secondary Roles ---

spanning-tree mode pvst

spanning-tree vlan 10 root primary

spanning-tree vlan 20 root secondary

! --- Step 3: Configure FastEthernet Trunk Interfaces ---

interface FastEthernet0/1

description Trunk to SW2 (Fa0/1)

switchport mode trunk

switchport trunk allowed vlan 10,20

spanning-tree guard root

no shutdown

exit

interface FastEthernet0/2

description Trunk to SW3 (Fa0/1)

switchport mode trunk

switchport trunk allowed vlan 10,20

spanning-tree guard root

no shutdown

exit

end

write memory

🏢 2. Switch 2 (SW2 — Primary Root for VLAN 20)

Plaintext

enable

configure terminal

hostname SW2

! --- Step 1: Create VLANs ---

vlan 10

name Sales_Data

exit

vlan 20

name Voice_Mgmt

exit

! --- Step 2: Configure PVST+ Primary & Secondary Roles ---

spanning-tree mode pvst

spanning-tree vlan 20 root primary

spanning-tree vlan 10 root secondary

! --- Step 3: Configure FastEthernet Trunk Interfaces ---

interface FastEthernet0/1

description Trunk to SW1 (Fa0/1)

switchport mode trunk

switchport trunk allowed vlan 10,20

spanning-tree guard root

no shutdown

exit

interface FastEthernet0/2

description Trunk to SW3 (Fa0/2)

switchport mode trunk

switchport trunk allowed vlan 10,20

spanning-tree guard root

no shutdown

exit

end

write memory

🏢 3. Switch 3 (SW3 — Access Switch with Global Security)

Plaintext

enable

configure terminal

hostname SW3

! --- Step 1: Create VLANs ---

vlan 10

name Sales_Data

exit

vlan 20

name Voice_Mgmt

exit

! --- Step 2: Global STP Security Configurations ---

spanning-tree mode pvst

spanning-tree portfast default

spanning-tree portfast bpduguard default

spanning-tree loopguard default

! --- Step 3: Configure Trunk Links Facing SW1 and SW2 ---

interface FastEthernet0/1

description Trunk to SW1 (Fa0/2)

switchport mode trunk

switchport trunk allowed vlan 10,20

spanning-tree portfast disable

no shutdown

exit

interface FastEthernet0/2

description Trunk to SW2 (Fa0/2)

switchport mode trunk

switchport trunk allowed vlan 10,20

spanning-tree portfast disable

no shutdown

exit

! --- Step 4: Configure Host Access Ports ---

interface FastEthernet0/3

description Access Link to PC-1

switchport mode access

switchport access vlan 10

no shutdown

exit

interface FastEthernet0/4

description Access Link to PC-2

switchport mode access

switchport access vlan 20

no shutdown

exit

end

write memory

- 🔬 Section 6: Verification & Diagnostic Toolkit

1. Verifying PVST+ Root Bridge Information per VLAN

Run these commands on SW3 to verify per-VLAN root paths:

Plaintext

SW3# show spanning-tree vlan 10

- What to look for: Root Bridge ID for VLAN 10 shows SW1's MAC address; Fa0/1 is the Root Port (FORWARDING).

Plaintext

SW3# show spanning-tree vlan 20

- What to look for: Root Bridge ID for VLAN 20 shows SW2's MAC address; Fa0/2 is the Root Port (FORWARDING).

2. Viewing STP Summary & Global Security Features

Plaintext

SW3# show spanning-tree summary

Sample Output:

Plaintext

Switch is in pvst mode

Root bridge for: none

Extended system ID is enabled

Portfast Default                  is enabled

PortFast BPDU Guard Default       is enabled

Loopguard Default                 is enabled

Configured Pathcost method v2     is short

Name                   Blocking Listening Learning Forwarding STP Active

---------------------- -------- --------- -------- ---------- ----------

VLAN0010                      1         0        0          2          3

VLAN0020                      1         0        0          2          3

---------------------- -------- --------- -------- ---------- ----------

2 vlans                       2         0        0          4          6

3. Checking Inconsistent Ports (Root Guard / Loop Guard Tripped)

If a rogue switch is plugged into SW1 or SW2's root-guarded ports:

Plaintext

SW1# show spanning-tree inconsistentports

Sample Output:

Plaintext

Name                 Interface              Inconsistency Reason

-------------------- ---------------------- ------------------

VLAN0010             FastEthernet0/2        Root Inconsistent

4. Recovering a BPDU Guard err-disable Port

If an unauthorized switch is plugged into Fa0/3 on SW3, BPDU Guard immediately shuts down the port.

- Check error state:

Plaintext

SW3# show interfaces status err-disabled

- Manual Recovery:

Plaintext

SW3(config)# interface FastEthernet0/3

SW3(config-if)# shutdown

SW3(config-if)# no shutdown

- Automatic Auto-Recovery Config (Optional):

Plaintext

SW3(config)# errdisable recovery cause bpduguard

SW3(config)# errdisable recovery interval 30

- ⚡ Section 7: Quick Memory Cheat Sheet for CCNA Exam Day

- Load Balance PVST+: spanning-tree vlan X root primary (Priority 24576)

- Enable PortFast Globally: spanning-tree portfast default (Non-trunking access ports)

- Enable BPDU Guard Globally: spanning-tree portfast bpduguard default

- BPDU Guard vs BPDU Filter: BPDU Guard disables port (err-disable). BPDU Filter on an interface silences STP completely (Extreme danger!).

- Root Guard: Configured on Designated Ports facing downstream switches → Prevents rogue root bridge takeovers (root-inconsistent).

- Loop Guard: Configured on Root / Alternate Ports → Protects against unidirectional link failure (loop-inconsistent).