Open Shortest Path First (OSPF) uses Cost as its metric to determine the best path to a destination. OSPF will always choose the path with the lowest cumulative cost.

If a modern network is left at default settings, OSPF becomes "blind" to high-speed links, causing poor routing decisions. This documentation outlines how OSPF calculates its metric, why the default formula fails today, and how to fix it.

- 1. The Core OSPF Metric Formula

OSPF calculates the cost of an individual interface using a fixed mathematical formula:

$$\text{Interface Cost} = \frac{\text{Reference Bandwidth}}{\text{Interface Bandwidth}}$$

- Reference Bandwidth: A global value configured on the router. By default, this is 100 Mbps ($10^8$ bits per second).

- Interface Bandwidth: The logical speed of the interface (e.g., FastEthernet = 100 Mbps, GigabitEthernet = 1000 Mbps).

⚠️ Important OSPF Rule: OSPF interface cost can never be a decimal or drop below 1. If the formula results in a number less than 1, the router automatically rounds up to 1.

- 2. The Modern OSPF "Blind Spot"

Because the default Reference Bandwidth was created in the 1990s when 100 Mbps was blazing fast, modern network interfaces suffer from a metric bottleneck.

Default Cost Comparison Table (Reference Bandwidth = 100 Mbps)

| Interface Type | Actual Speed | Formula Math | Final OSPF Cost |
| --- | --- | --- | --- |
| Ethernet | 10 Mbps | $100 \div 10 = 10$ | 10 |
| FastEthernet | 100 Mbps | $100 \div 100 = 1$ | 1 |
| GigabitEthernet | 1000 Mbps | $100 \div 1000 = 0.1 \to$ Round up | 1 (Blind Spot! ❌) |
| 10-Gigabit | 10,000 Mbps | $100 \div 10000 = 0.01 \to$ Round up | 1 (Blind Spot! ❌) |

The Problem

Because Gigabit and 10-Gigabit interfaces both share an identical OSPF cost of 1, OSPF cannot differentiate between a fast link and an ultra-fast link. This leads to Equal-Cost Multi-Path (ECMP) load balancing across sub-optimal paths.

- 3. Two Solutions for Traffic Engineering

To fix the blind spot or manually dictate traffic patterns, network engineers use two main approaches:

Approach A: The Global Fix (auto-cost reference-bandwidth)

This updates the reference bandwidth on all routers so the formula can scale up to modern speeds.

For a network utilizing up to 100 Gbps links, a reference bandwidth of 100,000 is recommended.

Plaintext

Router(config)# router ospf 1

Router(config-router)# auto-cost reference-bandwidth 100000

New Math with 100,000 Mbps Reference:

- FastEthernet (100 Mbps): $100,000 \div 100 =$ Cost 1000

- GigabitEthernet (1000 Mbps): $100,000 \div 1000 =$ Cost 100

- 10-Gigabit (10,000 Mbps): $100,000 \div 10,000 =$ Cost 10

🚨 Critical Rule: The reference bandwidth must match identically on every single router in the OSPF routing domain. If they do not match, different routers will compute different paths, causing routing loops.

Approach B: The Surgical Fix (ip ospf cost)

Instead of changing the global formula, you can hardcode the metric directly on a specific interface. This completely bypasses the mathematical formula and takes immediate priority.

Plaintext

Router(config)# interface gigabitEthernet 0/1

Router(config-if)# ip ospf cost 1000

Common Use Cases for Manual Cost:

- Primary/Backup Paths: Forcing traffic onto a low-latency premium fiber link while keeping an identical-speed backup line dormant.

- Graceful Maintenance: Cranking an interface cost to maximum (65535) to drain all live traffic off a wire before physically unplugging it for maintenance.

- Simulator Workarounds: Manually enforcing path behavior when virtual environments (like Cisco Packet Tracer) fail to update OSPF interface metrics dynamically after changing physical link speeds.

- 4. Day 22 Command Cheat Sheet

| Action | Command Mode | Command Syntax |
| --- | --- | --- |
| Change global OSPF scale | Router(config-router)# | auto-cost reference-bandwidth <Mbps> |
| Hardcode cost on an interface | Router(config-if)# | ip ospf cost <1-65535> |
| Adjust physical interface speed | Router(config-if)# | speed <10¦100¦1000> |
| Adjust logical OSPF bandwidth | Router(config-if)# | bandwidth <Kbps> |
| Force OSPF database recalculation | Router# | clear ip ospf process |
| View OSPF interface details & cost | Router# | show ip ospf interface <int> |
| Verify current OSPF routes | Router# | show ip route ospf |