A VLAN = logical broadcast domain inside a physical switch

Default: all ports = VLAN 1 (one big broadcast domain)

Creating a VLAN = building an invisible wall between groups of ports

VLAN   = Layer 2 concept (broadcast domain)

Subnet = Layer 3 concept (IP addressing)

Usually configured 1:1, but they are NOT the same thing

Proof: a device can have a correct IP for VLAN 10's subnet

but be physically assigned to VLAN 20 — it still won't

communicate with VLAN 10 devices

vlan 10                          → create VLAN 10

name Sales                       → name the VLAN

switchport mode access           → set port as access

switchport access vlan 10        → assign port to VLAN 10

switchport mode trunk            → set port as trunk

show vlan brief                  → see VLAN-to-port mapping

show interfaces trunk            → verify trunk status

vlan why?

Security  — isolate departments on same switch

Broadcast control — smaller broadcast domains

Flexibility — group by function, not physical location

Switch> enable

Switch# configure terminal

Switch(config)# vlan 10

Switch(config-vlan)# name Sales

Switch(config-vlan)# exit

Switch(config)# interface fastethernet 0/1

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

Switch(config-if)# exit

Switch(config)# interface fastethernet 0/3

Switch(config-if)# switchport mode trunk

Switch(config-if)# exit

Switch(config)# exit

Switch# write memory

Switch# show vlan brief

Switch# show interfaces trunk