Core Concept: The Broadcast Barrier

- The Problem: DHCP Discover messages are Layer 2 broadcasts. Routers, by design, discard broadcast traffic to prevent network congestion.

- The Solution: The ip helper-address command. It allows a router to act as a Relay Agent, intercepting the broadcast from the client and "relaying" it as a Unicast packet to a specific, remote DHCP server.

How It Works (The Relay Process)

- Intercept: The client sends a DHCP Discover broadcast. The Gateway router (Relay Agent) catches it on the interface connected to the client.

- Encapsulate: The router wraps this broadcast inside a Unicast packet.

- Address (giaddr): The router inserts its own interface IP into the giaddr (Gateway IP Address) field of the DHCP packet.

- Forward: The packet is sent across the network to the central DHCP Server.

- Assign: The server reads the giaddr to determine which subnet the request came from and assigns an IP from the corresponding DHCP pool.

Configuration Requirements

- Static IP: The DHCP Server (R1) must have a static IP address.

- Routing: There must be a valid path (static route or dynamic protocol like OSPF) between the Relay Agent and the DHCP Server.

- Interface Placement: The ip helper-address command must be applied to the interface directly connected to the DHCP clients.

Configuration Workflow (Based on your Topology)

1. On the Central Server (Router 0)

Define the pool for the remote subnet and ensure the gateway IP is excluded.

Plaintext

! Create the scope for the remote network

Router0(config)# ip dhcp pool REMOTE_NETWORK_20

Router0(dhcp-config)# network 192.168.20.0 255.255.255.0

Router0(dhcp-config)# default-router 192.168.20.1

Router0(dhcp-config)# dns-server 8.8.8.8

! Exclude the gateway so it isn't handed out to a PC

Router0(config)# ip dhcp excluded-address 192.168.20.1

2. On the Gateway Router (Router 1)

Apply the helper command to the interface facing the client (PC1).

Plaintext

! Point the relay agent to the IP of the DHCP server

Router1(config)# interface gig 0/1

Router1(config-if)# ip helper-address 192.168.90.1

Verification Checklist

- Verify Relay: Check that the interface is correctly configured to act as a relay.

Plaintext

Router1# show ip interface gig 0/1 | include Helper

- Verify Assignments: Check the server to see if it successfully leased an IP to the remote client.

Plaintext

Router0# show ip dhcp binding