- Theory: What is an Extended ACL?

- The Definition: An Extended Access Control List (ACL) is a highly precise traffic filter that inspects multiple layers of a packet at the exact same time.

- The Number Range: Extended ACLs use numbers 100–199 and 2000–2699 to tell the router to turn on advanced scanning.

- The 4-Point Inspection Checklist: Unlike a standard ACL (which only looks at where traffic came from), an Extended ACL checks four distinct things before making a decision:

  - Source IP: Who sent the packet?

  - Destination IP: Exactly which server or host is it trying to reach?

  - Protocol: Is it TCP (Web/SSH), UDP (DNS), ICMP (Ping), or IP (all traffic)?

  - Port Number: What specific application or service is it trying to use (e.g., HTTP port 80, HTTPS port 443)?

- 🧠 Why, When, & Where (The Core Logic)

❓ Why Do We Need It?

- Surgical Precision: Standard ACLs are a blunt axe; if you want to block a user from one webpage on a server, a Standard ACL forces you to block them from the entire server network. Extended ACLs are a scalpel; you can block just their web browsing while letting them access the same server's files, shared printers, and internal applications.

🕒 When Do We Use It?

- Use an Extended ACL whenever your security policy requires granular control based on applications or specific destinations (e.g., "Allow the accounting team to access the server via SSH, but block them from using Ping").

📍 Where Do We Place It?

- The Rule: Extended ACLs must always be placed as close to the source of the traffic as possible.

- The "Why": Because the Extended ACL already spells out the exact source and destination targets inside the rule, there is no guessing game. Dropping a banned packet right at its entrance interface prevents it from traveling across your WAN links, saving precious network bandwidth and router CPU cycles.

- ⚙️ How it Works under the Hood

- Top-to-Bottom Execution: The router evaluates your rules sequentially, starting at the lowest line number (usually line 10).

- First Match Wins: The absolute millisecond a packet matches the exact parameters of a line, the router executes the action (permit or deny) and instantly exits the list. It completely ignores any lines written below it.

- The Invisible Trapdoor: Every ACL has a hardcoded, unwritten deny ip any any (Implicit Deny All) at the absolute bottom. If a packet goes through the whole list without a single match, it gets dropped. You must include a permit statement at the end if you want other traffic to pass.

- 🛠️ Command Reference Guide (The How-To)

Step 1: Write the Filter Rules (On the Source Router)

- Block Web Traffic (HTTP Port 80):

Plaintext

Router(config)# access-list 100 deny tcp host [Source_IP] host [Dest_IP] eq 80

(Tells the router to drop TCP traffic traveling from a specific host to a specific host on port 80).

- Block Pings (ICMP):

Plaintext

Router(config)# access-list 100 deny icmp host [Source_IP] host [Dest_IP]

(Tells the router to drop all ping packets between these two hosts. Note: ICMP does not use port numbers, so no eq is needed).

- The Lifesaver Line:

Plaintext

Router(config)# access-list 100 permit ip any any

(Tells the router to allow all other types of traffic from any source to any destination, neutralizing the implicit deny wall).

Step 2: Bind it to the Interface

- Navigate to the interface:

Plaintext

Router(config)# interface [Interface_Name]

- Apply the policy inbound:

Plaintext

Router(config-if)# ip access-group 100 in

  - Why in? Because we place it close to the source, we want to catch the packet the moment it flies into the router's local gateway port.

Step 3: Verify and Diagnose

- Check hit counts and lines:

Plaintext

Router# show ip access-lists 100

- Verify which interface is running the filter:

Plaintext

Router# show ip interface [Interface_Name]

- ⚠️ Critical Troubleshooting Trap: Router-on-a-Stick (RoaS)

- The Physical Port Trap: If your topology uses a trunk link with subinterfaces (like GigabitEthernet0/0.10 and GigabitEthernet0/0.20), applying the ACL to the main physical interface (Gi0/0) will fail.

- The Fix: You must log into the specific subinterface acting as the default gateway for the target VLAN and apply the access-group there:

Plaintext

Router(config)# interface GigabitEthernet0/0.20

Router(config-subif)# ip access-group 100 in