Today, we moved beyond the "Router-on-a-Stick" model by using a Multilayer Switch. This is the standard method for routing traffic between VLANs in enterprise networks.

1. Why Layer 3 Switching is Important

- No Bottlenecks: In "Router-on-a-Stick," all inter-VLAN traffic must pass through a single physical trunk link. If that link gets congested, the whole network slows down.

- Wire-Speed Routing: A Multilayer switch performs routing using hardware (ASICs), not software. This means it routes traffic as fast as the ports can handle it.

- Cleaner Topology: You eliminate the need for an external router for internal traffic, reducing cable clutter and hardware costs.

2. The Core Concepts

- SVI (Switched Virtual Interface): A logical Layer 3 interface created for a specific VLAN. It acts as the "Default Gateway" for all devices within that VLAN.

- ip routing: This is the most critical command. By default, switches are Layer 2 only. This command enables the switch's internal routing engine, allowing it to behave like a router.

3. Essential Commands

Here is the configuration sequence you performed today:

- Enable the Routing Engine:

Plaintext

Switch(config)# ip routing

- Create VLANs:

Plaintext

Switch(config)# vlan 10

Switch(config-vlan)# vlan 20

- Configure the SVIs (The Gateways):

Plaintext

Switch(config)# interface vlan 10

Switch(config-if)# ip address 192.168.10.1 255.255.255.0

Switch(config)# interface vlan 20

Switch(config-if)# ip address 192.168.20.1 255.255.255.0

- Assign Ports to VLANs:

Plaintext

Switch(config)# interface fa0/1

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

4. Troubleshooting Summary

- Ping Failures: If PCs cannot ping, verify the Default Gateway on the PC settings matches the IP you assigned to the corresponding SVI on the switch.

- Routing Check: Run show ip route. If you don't see your VLAN subnets listed as "Connected," your routing is not active.

- Interface Status: Run show ip interface brief. If your interface vlan X is down/down, check if you have assigned at least one physical port to that VLAN and if that port is up.

This setup is the foundation of high-performance campus networking. By understanding how to move routing from a dedicated router to the switch core, you have learned how to build networks that are both faster and easier to manage.