Subnetting = way of breaking down a large network into smaller chuncks for better security

There is CIDR value like /24 , /26

Which are the total number of “1” out of 32.

Then there is host bits, which is 32-CIDR,

Now, number of ip addresses are found using, 2^n, among which there are questions 2 preused already, which means that 2^n – 2 are usuable ip address as the 1st one is used for netowkrs whereas last one is used for broadcast.

Once you get total number of ip addresses available  included network amd host bits

Eg: /26 cidr, then 32-26 = 6 this is host bits, now ip addresses = 2^n(eg: 6) = 2^6 = 64

Then the subnets will have the same ranges distribution.

So 1st subnet will be 0 63, 2nd =  subnet will be (64,127), 3rd(128-191), 4th (192-255)

Subnet 1: 192.168.1.0   - 192.168.1.63    network=.0    broadcast=.63

Subnet 2: 192.168.1.64  - 192.168.1.127   network=.64   broadcast=.127

Subnet 3: 192.168.1.128 - 192.168.1.191   network=.128  broadcast=.191

Subnet 4: 192.168.1.192 - 192.168.1.255   network=.192  broadcast=.255

Devices on different subnets cannot communicate without a router

Even a physical cable between switches is not enough — Layer 3 needed

Block Size

Block size = 256 - last octet of subnet mask

Subnets jump in multiples of block size

For /26: last octet = 192, block size = 256-192 = 64. That is why subnets jump at 0, 64, 128, 192.

Evidences: