- 1. Key Terms & Architectural Concepts

Dynamic Routing

- Definition: A system where routers use routing protocols to automatically share information about networks they can reach.

- Why Use It: Scalability and Self-Healing. It eliminates manual configuration in networks larger than 5–10 routers. If a link goes down, routers automatically calculate a detour path in milliseconds without human intervention.

Distance Vector vs. Link-State

Dynamic protocols are split into two primary operational behaviors:

- Distance Vector (e.g., RIP, EIGRP): Routes "by rumor." Routers only know the direction (vector) and distance (metric) to a destination based on what their immediate neighbors tell them. They do not know the full topology map.

- Link-State (e.g., OSPF): Routes by blueprint. Every router shares its local link statuses to build an identical, complete 3D topological map of the entire network before calculating paths.

Metric

- Definition: The cost value assigned to a route by a routing protocol to determine the "best path" when multiple paths exist to the same destination.

- Why Use It: Tie-breaker. RIP uses Hop Count (max 15 hops; 16 is unreachable). OSPF uses Cost (based on bandwidth).

Administrative Distance (AD)

- Definition: A rating of trustworthiness of a routing source (scale from 0 to 255). Lower numbers are more trusted.

- Why Use It: Allows a router to choose between different routing protocols. For example, a Static Route (AD 1) will always be chosen over a RIP route (AD 120).

- 2. RIPv2 Protocol Mechanics

- The 30-Second Update: RIPv2 routers copy their entire routing table and broadcast it out of all active interfaces every 30 seconds using the multicast address 224.0.0.9.

- Dead Timer: If a neighbor fails to send an update for 180 seconds, the router marks that neighbor's routes as invalid.

- 3. Configuration Commands Explained

Plaintext

Router(config)# router rip

Router(config-router)# version 2

Router(config-router)# no auto-summary

Router(config-router)# network 192.168.1.0

Command Breakdown:

- router rip

  - What it does: Enters the RIP protocol configuration mode.

- version 2

  - What it does: Forces the router to use RIP version 2 instead of the outdated version 1.

  - Why use it: Version 2 supports classless subnetting (VLSM) and secure multicast updates.

- no auto-summary

  - What it does: Disables automatic route summarization at classful boundaries.

  - Why use it: Prevents the router from accidentally converting specific networks (like 192.168.1.0/24) into standard classful blocks during updates, which breaks custom subnets.

- network [Classful Network ID]

  - What it does: Activates RIP on any local physical interface that owns an IP matching this network block.

  - Why use it: It tells the router to start sending/receiving RIP updates out of that physical port, and includes that interface's subnet inside the updates sent to neighbor routers.

- 4. Verification Commands

show ip rip database

- What it shows: The internal database of all routes learned or advertised via RIP.

- What to look for: Verify your local networks are listed as "directly connected" and remote subnets show up with their specific hop count inside brackets (e.g., [1]).

show ip route

- What it shows: The active routing table used by CEF to move traffic.

- What to look for: Look for lines starting with R. Ensure you see the bracketed metrics format [120/1], where 120 is the Administrative Distance and 1 is the Hop Count.

| Feature | Day 17: Static Routing | Day 18: RIPv2 Dynamic Routing |
| --- | --- | --- |
| Who maps the network? | You (The Human Engine). | The Routers (Automated Gossip). |
| What happens if a link dies? | Total failure until you manually re-type code. | Self-Heals automatically by finding a detour. |
| Config overhead? | Grows exponentially with every new router. | Stays simple; you only ever configure local ports. |
| Human Error Risk? | Extremely high (typos, loops). | Very low (routers handle the math). |