- Day 50 — Port Security Deep Dive

- 📌 Section 1: Core Mechanics (WHAT, WHY, WHEN, HOW)

🔹 What is Port Security?

- Port Security: A Layer 2 security feature acting as a "bouncer" at the edge of the network. It restricts input to an interface by limiting and identifying MAC addresses of the workstations allowed to access the port.

- WHY: Without it, anyone can walk into a building, plug a laptop into an open wall jack, and access the internal corporate network. Furthermore, hackers can launch MAC Flooding attacks to overwhelm the switch's memory, turning it into a vulnerable hub.

- WHEN: Applied to Access Ports (facing end-users, PCs, printers, and servers). Never applied to Trunk ports connecting switches together.

- HOW: By limiting the maximum number of allowed MAC addresses on a port and dictating exactly how the switch should punish unauthorized devices (Violation Modes).

- 🏢 Section 2: Real-World Scenarios

1. The "Hub" Problem

An employee needs more Ethernet ports, so they bring a cheap 5-port hub from home and plug it into the wall jack. Now, 4 unauthorized devices are feeding into a single switch port. Port Security (maximum 1) detects the second MAC address and instantly blocks it.

2. The MAC Flooding Attack

A hacker plugs in and uses a script to fire 10,000 fake MAC addresses per second into the switch. A switch's CAM (MAC) table can only hold about 8,000 entries. When full, the switch panics and broadcasts all traffic out of all ports—allowing the hacker to intercept data. Port Security stops this instantly at the very first fake MAC address.

- ⚔️ Section 3: BPDU Guard vs. Port Security (Exam Gold)

Cisco loves testing if you know the difference between these two edge-security features.

| Feature | The Target | The Trigger | The Goal |
| --- | --- | --- | --- |
| BPDU Guard (Day 48) | Rogue Switches | Receiving a BPDU frame | Prevent Spanning Tree (Layer 2) loops. |
| Port Security (Day 50) | Rogue PCs / Hackers | Unauthorized MAC Address | Prevent unauthorized network access and MAC floods. |

Note: In the real world, both are applied to access ports simultaneously.

- 🚨 Section 4: The Three Violation Modes (Memorize for Exam)

If an unauthorized MAC address is detected, the switch executes a penalty. You must know the exact behavior of all three modes:

| Violation Mode | Drops Bad Traffic? | Sends Log/Alert? | Increases Violation Counter? | Port State |
| --- | --- | --- | --- | --- |
| Shutdown (Default) | Yes | Yes | Yes | err-disabled (Port turns completely OFF / Red) |
| Restrict | Yes | Yes | Yes | UP (Port stays ON, bad traffic dropped, good traffic allowed) |
| Protect | Yes | NO | NO | UP (Port stays ON, ninja silent drop, no logs kept) |

- 📜 Section 5: Complete CLI Configuration Scripts

🏢 Enabling Port Security (The Standard Lockdown)

Plaintext

enable

configure terminal

interface FastEthernet0/1

! 1. Prerequisite: MUST be an access port (will fail on dynamic)

switchport mode access

! 2. Turn the feature ON

switchport port-security

! 3. Set the limit of allowed devices

switchport port-security maximum 1

! 4. Sticky MAC: Switch auto-learns and saves the first MAC it sees

switchport port-security mac-address sticky

! 5. Set the penalty (Shutdown is default, but good to hardcode)

switchport port-security violation shutdown

- 🔬 Section 6: Verification & Troubleshooting

1. The Switch's "Blind Spot"

When you plug a rogue PC in, the port stays UP initially. Switches do not have magic X-ray vision. They only detect electrical voltage when a cable is plugged in. The switch cannot read the MAC address and trigger the violation until the PC actually sends a frame (like a ping or DHCP request) onto the wire.

2. The Crime Scene Command (Check the Bouncer)

Plaintext

SW1# show port-security interface fa0/1

- Port Security: Enabled

- Port Status: Secure-down (if Shutdown) or Secure-up (if Restrict/Protect)

- Violation Mode: Shutdown / Restrict / Protect

- Security Violation Count: Tracks exactly how many illicit packets were blocked. (Will be 0 if using Protect).

3. Recovering an Err-Disabled Port (Manual vs Auto)

Once a port hits err-disable, you must unplug the rogue device, or it will just crash again.

Manual Recovery:

Plaintext

SW1(config)# interface FastEthernet0/1

SW1(config-if)# shutdown

SW1(config-if)# no shutdown

Auto-Recovery (The Enterprise Cheat Code):

Tells the switch to automatically attempt to turn the port back on after a set timer.

Plaintext

SW1(config)# errdisable recovery cause psecure-violation

SW1(config)# errdisable recovery interval 300

(Timer is in seconds, so 300 = 5 minutes).

- ⚡ Section 7: Quick Memory Cheat Sheet for Exam Day

- Prerequisite: Port must be statically set to switchport mode access.

- Sticky: mac-address sticky saves you from manually typing out hex MAC addresses. It saves the learned MAC into the running-config.

- Shutdown: Kills port, creates logs.

- Restrict: Leaves port up, drops bad traffic, creates logs & counts them.

- Protect: Leaves port up, drops bad traffic, no logs, no counts (silent).

- Recovery: Must bounce the port (shut / no shut) to clear an err-disable state.