- 1. What is an ACL?

An Access Control List (ACL) is a sequential list of conditions (an if/else block) used by a router to classify network traffic.

- The Primary Use: It acts as a firewall bouncer to Permit (allow) or Deny (drop) packets.

- Other Uses: It can select traffic for NAT translation or prioritize traffic for QoS (Quality of Service).

- ⚙️ 2. How Does a Standard ACL Work?

- Single-Lens Focus: Standard ACLs (numbered 1–99) are blind to destination IPs and ports. They only look at the Source IP Address ("Where did you come from?").

- Top-to-Bottom: The router evaluates lines sequentially (Line 10, then 20, then 30).

- First Match Wins: The absolute instant a packet matches a line, the router executes the action (permit/deny) and stops reading.

- The Implicit Deny All: Every ACL has an invisible deny any at the very bottom. If a packet matches nothing, it gets dropped. Every valid ACL needs at least one permit statement.

- 📍 3. The Placement Rule: Why Close to the Destination?

Rule: Standard ACLs must always be placed as close to the destination network as possible.

- The "Why": Because a Standard ACL can only see the source IP, it is a blunt weapon.

- If you place it close to the source, it will block that source from reaching everything on the network (including the Internet).

- By placing it right outside the destination network's front door, you protect that specific network while allowing the source to communicate normally with the rest of the world.

- 🧮 4. Wildcard Masks Cheat Sheet

Wildcard masks are the inverse of subnet masks. 0 = Match exactly; 255 = Ignore.

- Target a single host: 0.0.0.0 (or use the keyword host)

- Target a full /24 subnet: 0.0.0.255

- Target everyone: any

- 🛠️ 5. Clean Command Cheat Sheet

Pre-Requisite (Crucial):

Ensure OSPF or static routing is running and all pings succeed before configuring the ACL.

Step 1: Create the Rules (On the Destination Router)

Plaintext

Router(config)# access-list 1 deny host 192.168.20.50

Router(config)# access-list 1 permit 192.168.10.0 0.0.0.255

Router(config)# access-list 1 permit any

Step 2: Glue it to the Interface (Outbound)

Plaintext

Router(config)# interface FastEthernet 0/0

Router(config-if)# ip access-group 1 out

Step 3: Verify and Track Hits

Plaintext

Router# show ip access-lists

Router# show ip interface FastEthernet 0/0

- The Real-World Scenario: The Over-Curious Intern

Imagine you are the Network Administrator for a corporate headquarters. You have an intern machine in the Engineering Department, a shared Cafeteria Printer, and a highly sensitive HR Payroll Server.

Plaintext

[Intern PC] (Source)

192.168.50.21

│

▼

[ R1 ] ─────── (Fa0/1) ───────► [Cafeteria Printer] (192.168.1.5)

│

(Gi0/0)

│

▼

[ R2 ] ─────── (Fa0/0) ───────► [HR Payroll Server] (10.10.10.100) (Destination)

- The Goal: Block the Intern PC (192.168.50.21) from ever accessing the HR Payroll Server (10.10.10.100).

- The Condition: The intern still needs to print documents in the Cafeteria and access the internet.

❌ What Happens If You Place It Close to the Source? (Bad Design)

You decide to create a Standard ACL on R1 and apply it Inbound on the interface where the intern plugs in.

Plaintext

Router1(config)# access-list 1 deny host 192.168.50.21

Router1(config)# access-list 1 permit any

Router1(config)# interface FastEthernet 0/0

Router1(config-if)# ip access-group 1 in

The Result: The intern tries to print a document. The packet enters R1. The Standard ACL looks at the source: "Ah, you are 192.168.50.21. Line 10 says Deny!" Boom. Packet dropped. The intern can't reach the HR server, but they also can't print, and they can't get to the internet. Because the Standard ACL can't see the destination, it treats every packet from the intern the exact same way. You have accidentally completely cut them off from the entire company network.

What Happens If You Place It Close to the Destination? (Correct Design)

Instead, you delete that ACL and move down to R2, applying it Outbound on the interface directly facing the HR Server (Fa0/0).

Plaintext

Router2(config)# access-list 1 deny host 192.168.50.21

Router2(config)# access-list 1 permit any

Router2(config)# interface FastEthernet 0/0

Router2(config-if)# ip access-group 1 out

The Result: 1. Intern prints a document: The packet hits R1, looks at the destination (Printer), and R1 routes it out of Fa0/1. The packet never hits R2. Printing works perfectly! 2. Intern tries to hack HR: The packet hits R1, gets routed to R2. R2 looks at its routing table and sees the HR server lives out of Fa0/0. Right before the packet flies out onto the wire, the ACL bouncer stops it: "Wait, your source is 192.168.50.21. You are banned from this specific door." Packet dropped.

By putting the bouncer at the destination's front door, you protected HR without ruining the intern's ability to do their job elsewhere.