- Lab Notes: Day 53 - NTP and Syslog

1. NTP (Network Time Protocol)

- Why we need it: If network devices have different times, correlating logs during a cyber attack or network outage is impossible. NTP ensures every device matches exactly.

- Stratum Levels: Defines the distance from the highly accurate reference clock.

  - Stratum 0 = Atomic/GPS clock.

  - Stratum 1 = Server connected to Stratum 0.

  - Stratum 2 = Device syncing to a Stratum 1 server.

- Configuration & Verification:

- Router(config)# ntp server 192.168.10.100

- Router# show ntp status       <-- Verifies if the clock is synchronized and shows the Stratum.

- Router# show ntp associations <-- Shows the servers the router is talking to.

- Pro-Tip: NTP is a slow protocol. It takes several minutes to synchronize. In Packet Tracer, use the "Fast Forward Time" button to speed this up.

2. Syslog (Centralized Logging)

- Why we need it: Devices generate logs for everything (interfaces dropping, routing changes). Sending them to a central Syslog Server creates a permanent, searchable database.

- Configuration:

- Router(config)# logging host 192.168.10.100

- Router(config)# logging trap informational  <-- Sets the severity threshold

3. Syslog Severity Levels (0-7)

The logging trap command tells the router to send logs of the specified level and all lower numbers (more critical). (Memory Trick: Every Awesome Cisco Engineer Will Need Ice Cream)

- 0 - Emergency: System unusable

- 1 - Alert: Immediate action needed

- 2 - Critical: Critical conditions

- 3 - Error: Error conditions

- 4 - Warning: Warning conditions

- 5 - Notification: Normal but significant conditions

- 6 - Informational: Informational messages (Standard baseline for monitoring)

- 7 - Debug: Debugging messages (Massive amounts of data, use carefully!)

4. The Golden Rule of Logging

Logs are useless if they just show "Uptime" (e.g., 00:14:33). You MUST tell the router to stamp logs with the synchronized NTP date and time.

Router(config)# service timestamps log datetime msec