If 1 pc is not pinging to other check

Layer 1 st

Then layer 2  vlan -> trunk ->  STP -> ether channel

Bug 1 : different vlan

How to check: show vlan brief on both switches and see if the pc’s trying to ping are both in same vlan.

Bug 2 : native vlan mismatch

IN trunk of 2 switches port of 1st  switch is prompted for one native vlan, however 2nd switch port connecting to 1st switch is on different native vlan.

This difference in native vlan doesn’t make a huge impact or it wont be showed.

Native vlan on trunk is for when the packet sent is untagged meaning packet is sent with  no vlan configuration.

Meaning the switches can still communicate the tagged vlan packets only untagged packet gets lossed.

Bug 3: STP priority missing

STP: spanning tree protocol: when 3 or more switches are connected to eachother like a triangle then there might be a switching loop.

It elects a Root Bridge based on Bridge Priority (and MAC address as a tiebreaker). After root selection, STP determines port roles (root port, designated port, blocked port).

If all switches have default priority, STP still functions correctly using MAC address comparison. However, without manually setting bridge priorities, the root bridge placement may be suboptimal, which can lead to inefficient paths and temporary instability during topology changes.

Lowest bridge value is 4096. So when we set it to that when the switches are compared it gets selected cause its smallaest. If 2 switches has 4096 priority then mac address is compared.

Bug 4: trunk port change to access

If there is only 1 path between 2 switches it must be on trunk to allow communication. However if 2 paths are there 1 path set to access is only usuable for the access vlan it is set.

So even if trunk is changed to access the root bridge remains same.

Bug 5: etherchannel mode passive on both switches it wont work.

EtherChannel with LACP requires at least one side in active mode. Passive + Passive never forms because neither side initiates negotiation.

Bug 6: vtp domain mismatch

If the vtp domain name mistakenly mismatched vlan isn’t shown.

.