- The Network Engineer's Troubleshooting Hack Guide

- The Golden Rule of Troubleshooting: Follow the Packet

Never guess. Prove exactly where the packet dies using targeted show commands.

- Layer 1/2 (Switching): Can the PC talk to the Switch?

- Layer 3 Local (Gateway): Can the PC talk to its Default Gateway (the Router)?

- Layer 3 Remote (Routing): Can the local Router talk to the remote Router?

- Layer 4/7 (Security): Is an ACL or NAT rule blocking the traffic?

- 📖 The CLI Keyword Cheat Sheet

When you run show commands, here is what the router/switch is secretly trying to tell you:

- err-disabled (in show int status): The switch panicked. A security rule (Port Security, BPDU Guard) was violated, and the switch turned the port off to protect the network.

- unassigned (in show ip int brief): The interface is turned on, but you forgot to give it an IP address. It cannot route traffic.

- up / down (in show ip int brief): Layer 1 (Cable) is plugged in, but Layer 2 (Protocol) is failing. Often caused by mismatched encapsulation, speed, or duplex.

- FULL (in show ip ospf neighbor): Perfection. The two routers have successfully exchanged their routing tables.

- INIT or EXSTART (in OSPF): The routers see each other, but they are stuck arguing over MTU size or passwords.

- Empty Table (in OSPF): The routers are completely ignoring each other. Usually caused by a wrong network statement or mismatched subnet masks.

- 🪲 Day 58 Gauntlet: Detailed Bug Breakdown

BUG 1: The Port Security Lockdown (Layer 2)

- Context: Switch ports should only allow authorized PCs. We configured Port Security to memorize the first PC plugged in (sticky MAC).

- The Symptom: PC couldn't pull DHCP. Port light was red.

- How to Deduce & Prove it:

  - Run show interface fa0/10 status. Saw err-disabled. Deduction: Switch blocked the port intentionally.

  - Run show port-security interface fa0/10. Saw Secure-down and a violation.

  - Run show run | begin interface fa0/10. Saw a hardcoded fake MAC (0000.1111.2222).

- The Fix: Remove the fake MAC, apply mac-address sticky, and "reboot" the port using shutdown then no shutdown.

BUG 2: The Native VLAN Mismatch (Layer 2 Trunking)

- Context: Trunks carry multiple VLANs by "tagging" them. The Native VLAN is the designated lane for untagged (management/CDP) traffic. Both ends of the wire MUST agree on which VLAN is the Native one.

- The Symptom: Network worked for data (tagged PC traffic successfully reached the gateway), but the switch console spammed %CDP-4-NATIVE_VLAN_MISMATCH errors.

- How to Deduce & Prove it:

  - On the Switch: show interfaces trunk. Look at the Native VLAN column. (It said 1).

  - On the Router: show interfaces gig0/0.999. (It said Encapsulation 802.1Q ... Native VLAN 999).

  - Deduction: The switch is sending untagged traffic on VLAN 1; the router expects it on 999.

- The Fix: On the Switch trunk port, run switchport trunk native vlan 999 to match the router.

BUG 3: The Missing Gateway IP (Layer 3)

- Context: For Inter-VLAN routing (Router-on-a-Stick), every VLAN needs a virtual sub-interface on the router to act as its Default Gateway.

- The Symptom: A PC in VLAN 200 (Marketing) couldn't ping anything. The router itself couldn't ping the VLAN 200 gateway.

- How to Deduce & Prove it:

  - Run show ip interface brief on R_CORE.

  - Look at Gig0/0.200. The status was up, but the IP Address column said unassigned.

  - Deduction: The sub-interface was created, but the IP address command was forgotten.

- The Fix: Go into interface gig0/0.200 and apply ip address 172.16.2.1 255.255.255.0.

BUG 4: The OSPF Network Statement Typo (Layer 3 Routing)

- Context: OSPF dynamically builds routing tables. A router only participates in OSPF on interfaces that match its network statements.

- The Symptom: PC could ping its Gateway (172.16.1.1), but could NOT ping the next-hop Edge Router (10.10.10.1).

- How to Deduce & Prove it:

  - Run show ip ospf neighbor on R_CORE. The table was completely empty.

  - Run show ip protocols on R_CORE. Looked under "Routing for Networks". It said 10.10.20.0 0.0.0.3.

  - Run show ip interface brief on R_CORE. The interface connecting to the Edge router was 10.10.10.2.

  - Deduction: R_CORE was trying to send OSPF Hello packets out to a 10.10.20.0 network that didn't exist, completely ignoring the actual Gig0/1 interface.

- The Fix: Enter router ospf 10, remove the bad network (no network 10.10.20.0...), and add the correct one (network 10.10.10.0 0.0.0.3 area 0).

- 🧠 The Senior Engineer Mindset Check

Why did the ping to the gateway work in Bug 2, even though the trunk was mismatched? Because the PC's traffic was mapped to VLAN 100. The switch tagged it with a 100, and sent it up the trunk. The router read the 100 tag and routed it. The Native VLAN only affects untagged frames. We fix it not to restore data flow, but to restore network stability (CDP and STP).

What happens if a ping succeeds on the way out, but fails on the way back? Routing is a two-way street. If R_CORE knows how to reach R_EDGE, the ping gets there. But if R_EDGE doesn't have an OSPF route back to the 172.16.1.0 network, it drops the reply. Always check show ip route on both ends!