- Day 46 — IPv6 Neighbor Discovery Protocol (NDP)

- 📌 Section 1: Core Concepts (WHAT, WHY, WHEN, HOW)

🔹 What is NDP?

- WHAT: Neighbor Discovery Protocol (NDP) is an ICMPv6-based suite of messaging functions that handles Layer 3-to-Layer 2 mapping, gateway discovery, and IP collision avoidance in IPv6 networks.

- WHY: IPv4 relies heavily on Address Resolution Protocol (ARP) and Layer 2 Broadcasts (FF-FF-FF-FF-FF-FF), which forces every single CPU on a switch segment to process every ARP request. IPv6 completely eliminates broadcasts in favor of targeted ICMPv6 multicast, making the network exponentially more efficient.

- WHEN: NDP runs automatically on every IPv6-enabled interface as soon as an address is configured or ipv6 unicast-routing is enabled.

- HOW: It uses four primary ICMPv6 message types and special Solicited-Node Multicast groups to directly address target NICs.

- ✉️ Section 2: The 4 ICMPv6 Messages (How to Distinguish)

NDP replaces multiple IPv4 protocols (ARP, ICMP Redirects, Router Discovery) using four core ICMPv6 message types.

| Message Name | ICMPv6 Type | Direction | Target Destination Address | IPv4 Equivalent |
| --- | --- | --- | --- | --- |
| Router Solicitation (RS) | 133 | PC →Router | FF02::2 (All-Routers Multicast) | DHCP Discover / Router Discovery |
| Router Advertisement (RA) | 134 | Router →PC / All | FF02::1 (All-Nodes Multicast) or Unicast | DHCP Offer / Default Gateway Announcement |
| Neighbor Solicitation (NS) | 135 | Host →Target Host | FF02::1:FFXX:XXXX (Solicited-Node Multicast) | ARP Request (also used for DAD) |
| Neighbor Advertisement (NA) | 136 | Target Host →Host | Unicast (or FF02::1 during DAD) | ARP Reply |

💡 How to Distinguish Message Functions at a Glance

- Router Messages (RS / RA = Types 133 / 134):

  - Used between Clients and Routers.

  - RS (133): The client asks, "Where is my default gateway?"

  - RA (134): The router replies, "I am your gateway, here is the network prefix and SLAAC rules!"

- Neighbor Messages (NS / NA = Types 135 / 136):

  - Used between Host and Host (or Host and Router's interface directly).

  - NS (135): Host asks, "Who has this specific IPv6 address? What is your MAC address?"

  - NA (136): Target replies, "I have that IPv6 address, here is my MAC address!"

- 🔬 Section 3: Step-by-Step NDP Operational Workflows

1. MAC Address Resolution (Replacing ARP)

Plaintext

[ PC-1 ] --------------------> ( Switch ) --------------------> [ PC-2 ]

(2001:db8:1::10)        Sends NS (Type 135) to               (2001:db8:1::20)

FF02::1:FF00:20 Multicast

[ PC-1 ] <-------------------- ( Switch ) <-------------------- [ PC-2 ]

Received MAC           Unicasts NA (Type 136) back

containing MAC 0060.2f22.2222

- Trigger: PC-1 wants to ping PC-2 (2001:DB8:1::20), but only knows its IPv6 address.

- NS Sent: PC-1 sends a Neighbor Solicitation (Type 135) to PC-2's Solicited-Node Multicast Address (FF02::1:FF00:20).

- Filtering: The switch floods the multicast frame, but only PC-2's NIC listens to FF02::1:FF00:20. Other hosts drop the frame at the hardware level without waking up their CPU.

- NA Reply: PC-2 replies directly to PC-1 with a Unicast Neighbor Advertisement (Type 136) containing its MAC address (0060.2F22.2222).

- Caching: PC-1 stores PC-2's MAC address in its IPv6 Neighbor Table.

2. Duplicate Address Detection (DAD)

Before an IPv6 address can be activated on an interface (whether static or via SLAAC), the device must verify that no other host is using it:

- Tentative State: The new IPv6 address is placed in a Tentative state.

- NS Probe: The host sends an NS (Type 135) with a unspecified source IP (::) asking: "Who has this IPv6 address?"

- Resolution:

  - If no NA is received: The address is unique and transitions to Active.

  - If an NA (Type 136) is received: A collision is detected, and the interface disables the duplicate IP.

- 📊 Section 4: Neighbor Table States & Diagnostic Logic

To view learned Layer 3-to-Layer 2 mappings on a Cisco router, use show ipv6 neighbors (replaces IPv4's show arp).

Neighbor Cache States (show ipv6 neighbors)

| State | Meaning | What is Happening Under the Hood? |
| --- | --- | --- |
| INCMP (Incomplete) | Address resolution in progress | An NS message was sent, but no NA reply has been received yet. |
| REACH (Reachable) | Active and confirmed | Traffic was sent recently and reachability was verified. |
| STALE | Idle for > 30 seconds | The MAC address is still cached, but the reachability timer expired. It stays here until new traffic is sent. |
| DELAY | Transitioning state | Traffic is being sent to a STALE neighbor; waiting briefly for positive confirmation. |
| PROBE | Actively re-verifying | Router is sending periodic unicast NS messages to confirm the neighbor is still online. |

⚠️ Common Troubleshooting Trap: "Why is show ipv6 neighbors Empty?"

- THE ISSUE: You enter show ipv6 neighbors on R1 immediately after configuring it, and the output is blank.

- WHY: IPv6 neighbor tables are built strictly on demand. If R1 has not transmitted data to PC-1 or PC-2, it has no reason to store their MAC addresses.

- HOW TO FIX: Send a ping from R1 to the target IP first to force NDP address resolution, then run show ipv6 neighbors again.

- 🔀 Section 5: "When to Use What" Command Reference

IPv4 vs. IPv6 Command Mapping

| Task | IPv4 Command | IPv6 Equivalent Command |
| --- | --- | --- |
| Check MAC Address Mapping Table | show arp | show ipv6 neighbors |
| View Interface Address Details | show ip interface brief | show ipv6 interface brief |
| View Joined Multicast Groups & DAD | N/A | show ipv6 interface <interface-id> |
| Check PC Neighbor Table (Windows) | arp -a | netsh interface ipv6 show neighbors |
| Enable Routing Engine | Enabled by default | ipv6 unicast-routing (Global Config) |

- 🛠️ Section 6: Complete CLI Cheat Sheet

Router Verification Commands

- View Neighbor Table:

Plaintext

R1# show ipv6 neighbors

Use when: You need to verify if the router has resolved the MAC address of a local PC or adjacent router.

- Inspect NDP Group Memberships & DAD Status:

Plaintext

R1# show ipv6 interface GigabitEthernet 0/1

Use when: Checking if an interface joined FF02::1 (All-Nodes), FF02::2 (All-Routers), or its Solicited-Node Multicast address (FF02::1:FFXX:XXXX).

- Clear Neighbor Cache (Force Re-resolution):

Plaintext

R1# clear ipv6 neighbors

Use when: Troubleshooting hardware swap issues or stale MAC entries.

PC Command Prompt Diagnostic Commands

- Display IPv6 Address & Gateway:

DOS

ipconfig /all

- View PC IPv6 Neighbor Cache:

DOS

netsh interface ipv6 show neighbors

- Flush PC Neighbor Cache:

DOS

netsh interface ipv6 delete neighbors