VLAN: vlan 10 / name HR / exit switchport mode access switchport access vlan 10 TRUNK: switchport mode trunk switchport trunk native vlan 99 switchport trunk allowed vlan all VTP: vtp domain NAME vtp mode server | client | transparent STP: spanning-tree mode rapid-pvst spanning-tree vlan 1-4094 priority 4096 (every VLAN, always) spanning-tree portfast spanning-tree bpduguard enable ETHERCHANNEL: interface range fa0/4 - 5 channel-group 1 mode active (LACP) interface port-channel 1 switchport mode trunk VERIFY (run before every test): show vlan brief show interfaces trunk show vtp status show spanning-tree show etherchannel summary show mac-address-table

1. Layer 1 show interfaces fa0/x status (connected? notconnect? err-disabled?) 2. VLAN show vlan brief (right port, right VLAN?) 3. Trunk show interfaces trunk (trunking? native matches both ends?) 4. STP show spanning-tree (correct root? unexpected BLK?) 5. EtherChan show etherchannel summary (SU status? both ports P?) Then, and only then, run ping tests.

- Network Configuration & Verification Workflow

- 🔹 VLAN Configuration

bash

vlan 10

name HR

exit

!

interface fa0/x

switchport mode access

switchport access vlan 10

- 🔹 Trunk Configuration

bash

interface fa0/x

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan all

- 🔹 VTP Configuration

bash

vtp domain NAME

vtp mode {server | client | transparent}

- 🔹 STP Configuration

bash

spanning-tree mode rapid-pvst

spanning-tree vlan 1-4094 priority 4096

spanning-tree portfast

spanning-tree bpduguard enable

- 🔹 EtherChannel (LACP)

bash

interface range fa0/4 - 5

channel-group 1 mode active

!

interface port-channel 1

switchport mode trunk

- 🔹 Verification Commands (Run before every test)

bash

show vlan brief

show interfaces trunk

show vtp status

show spanning-tree

show etherchannel summary

show mac-address-table

- ✅ Troubleshooting Workflow (Layer by Layer)

1. Layer 1 – Physical

bash

show interfaces fa0/x status

- Check if ports are connected, notconnect, or err-disabled.

2. VLAN

bash

show vlan brief

- Verify correct port assignment to the right VLAN.

3. Trunk

bash

show interfaces trunk

- Confirm trunking is active.

- Ensure native VLAN matches on both ends.

4. STP

bash

show spanning-tree

- Verify root bridge is correct.

- Check for unexpected blocked (BLK) ports.

5. EtherChannel

bash

show etherchannel summary

- Look for SU status (Layer2, in use).

- Ensure both ports show P (in port-channel).

6. Connectivity Test

- Only after all above checks pass:

bash

ping <destination>