- Wildcard Mask Essentials

- Wildcard Mask: An inverse subnet mask where 0 means "must match exactly" and 255 means "ignore this bit."

- The Formula: Calculate any wildcard mask by subtracting the standard subnet mask from 255.255.255.255.

- Common Examples: A /24 subnet mask becomes 0.0.0.255, and a point-to-point /30 WAN mask becomes 0.0.0.3.

- 🛠️ Configuration Commands

- router ospf [Process ID]: Enables OSPF locally using a tracking number that does not need to match your neighbor.

- network [IP] [Wildcard] area [ID]: Enables OSPF on any local interface whose IP address falls within the specified block.

- area 0: Binds the matching physical interface directly to the mandatory, loop-free OSPF backbone area.

- 🔬 Field Verification Diagnostics

- show ip ospf neighbor: Shows your active OSPF connections; the state column must read FULL for successful routing.

- Dead Time Countdown: A safety timer (defaults to 40s) that deletes the neighbor's routes if it reaches zero.

- show ip ospf interface [port]: Displays link-level details, including the network type, interface cost, and active timers.

- Timer Mismatch Rule: Hello (10s) and Dead (40s) timers must match exactly on both sides of the cable or OSPF breaks.

- show ip route: Displays the final forwarding table; look for the letter O to verify routes learned dynamically via OSPF.