- Day 49 — EtherChannel Deep Dive & Troubleshooting

- 📌 Section 1: Core Mechanics (WHAT, WHY, WHEN, HOW)

🔹 What is EtherChannel?

- EtherChannel (Port-Channel / Link Aggregation): A technology that bundles multiple physical Ethernet links into a single logical (virtual) link.

- WHY: By default, Spanning Tree Protocol (STP) blocks redundant physical cables between switches to prevent loops, wasting 50% or more of your bandwidth. EtherChannel tricks STP into seeing the bundle as one single cable, allowing all links to forward traffic simultaneously.

- WHEN: Used anywhere you need high bandwidth and instant fault tolerance between switches, routers, or servers.

- HOW: Configured using negotiation protocols (LACP, PAgP) to safely bind the ports together and manage traffic distribution via hashing algorithms.

- 🏢 Section 2: Real-World Scenario — The Bandwidth Pool

The "Trapped Traffic" Problem

- Without EtherChannel (Using PVST+): VLAN 10 gets one 1Gbps cable, and VLAN 20 gets the other. If VLAN 10 needs 1.5Gbps for a database backup, it chokes and drops packets, even if VLAN 20's cable is completely empty. The bandwidth cannot be shared.

- With EtherChannel: The two 1Gbps cables become a single 2Gbps superhighway. Both VLAN 10 and VLAN 20 share the massive pipe. If one VLAN is quiet, the other can consume the entire 2Gbps.

- The Failover Speed: If a physical wire inside an EtherChannel gets cut, STP does not recalculate. The switch hardware instantly shifts all traffic to the surviving wires in milliseconds. Users experience zero downtime.

- 🔑 Section 3: The Three Protocols & Load Balancing

1. The Negotiation Protocols (Memorize for Exam)

| Protocol | Standard | Keywords | Best Use Case |
| --- | --- | --- | --- |
| LACP | 802.3ad (Open Standard) | active / passive | Always use this. Works across all vendors (Cisco, HP, Dell). |
| PAgP | Cisco Proprietary | desirable / auto | Legacy networks with exclusively older Cisco switches. |
| Static | None (Hardcoded) | on | Connections to dumb devices. ⚠️ Extreme Risk: Skips safety checks and causes instant loops if misconfigured. |

2. The Load Balancing Algorithm (The Hash)

EtherChannel doesn't send packets round-robin. It runs a math equation (hash) to assign traffic flows to specific physical wires to keep frames in order.

- src-mac: (Default) Traffic from PC-1 always goes down Wire A. Traffic from PC-2 goes down Wire B.

- dst-mac: Based on the destination's MAC address.

- src-dst-ip: (The Best Method). The switch looks at both the Source and Destination IP addresses and runs an XOR calculation. This creates the most randomized, perfectly even 50/50 split of traffic across all wires.

- 🗺️ Section 4: Lab Topology Architecture

Plaintext

+----------------+

|      SW2       |

| (LACP Partner) |

+-------+--------+

|

Fa0/1, Fa0/2 (Po1)

|

+----------------+             +-------+--------+             +----------------+

|      SW3       +-------------+      SW1       +-------------+      SW4       |

| (PAgP Partner) | Fa0/3, 4    |  (Core Switch) | Fa0/5, 6    | (Static Partner|

+----------------+ (Po2)       +----------------+ (Po3)       +----------------+

- 📜 Section 5: Complete CLI Configuration Scripts

🏢 SW1 (The Core Switch — Configuring All 3 Types)

Plaintext

enable

configure terminal

hostname SW1

! --- 1. LACP Bundle to SW2 (The Standard) ---

interface range FastEthernet0/1 - 2

switchport mode dynamic desirable

channel-group 1 mode active

exit

! --- 2. PAgP Bundle to SW3 (Legacy Cisco) ---

interface range FastEthernet0/3 - 4

switchport mode dynamic desirable

channel-group 2 mode desirable

exit

! --- 3. Static Bundle to SW4 (Hardcoded) ---

interface range FastEthernet0/5 - 6

switchport mode dynamic desirable

channel-group 3 mode on

exit

! --- 4. Optimize the Load Balancing Algorithm ---

port-channel load-balance src-dst-ip

exit

write memory

🧠 The "Golden Rule" of Port-Channels

Once you type channel-group X, a virtual interface is created. From that moment on, you must apply all Trunk and VLAN settings to the interface port-channel X NOT the physical ports. The physical ports will automatically inherit the boss's (the Port-Channel's) instructions.

- 🔬 Section 6: Verification Toolkit

1. The Quick Summary (Your Best Friend)

Plaintext

SW1# show etherchannel summary

- What to look for:

  - (SU) next to the Port-Channel: S = Layer 2 (Switched), U = In Use. (This is perfection).

  - (SD) next to the Port-Channel: S = Layer 2, D = Down. (Bundle has crashed).

  - (P) next to the Physical Port: Bundled in port-channel.

  - (s) next to the Physical Port: Suspended. (Port mismatch detected).

2. The Deep Detail (If summary fails)

Plaintext

SW1# show etherchannel port-channel

- Note: In real Cisco gear, you use show etherchannel detail, but Packet Tracer requires the port-channel keyword.

- What to look for: Verifies the exact protocol being used and shows the Active or Desirable state of the internal ports.

3. Verify the Hash Algorithm

Plaintext

SW1# show etherchannel load-balance

- What to look for: Confirms the Operational State has changed to src-dst-ip for perfect distribution.

- 💥 Section 7: Troubleshooting the "Suspended" Error

EtherChannel is incredibly strict. If physical ports do not perfectly match in Speed, Duplex, Trunk Mode, DTP Status, or Allowed VLANs, the switch will throw this error:

%EC-5-CANNOT_BUNDLE2: Fa0/1 is not compatible with Fa0/2 and will be suspended

The "Ghost Interface" Bug & The Nuclear Fix

Sometimes, after removing misconfigured ports from a bundle, the virtual Port-Channel interface holds onto the corrupted configuration in the background (a ghost interface). When you put the cables back in, it instantly crashes them again.

The Nuclear Sledgehammer Fix:

- Remove the physical cables from the group.

- Destroy the ghost virtual interface entirely.

- Wipe the physical ports back to absolute factory default.

- Rebuild it cleanly from scratch.

Plaintext

SW1# configure terminal

! 1. Empty the group

SW1(config)# interface range FastEthernet0/1 - 2

SW1(config-if-range)# no channel-group 1 mode active

SW1(config-if-range)# exit

! 2. Kill the ghost interface

SW1(config)# no interface port-channel 1

! 3. Factory wipe the physical ports

SW1(config)# default interface range FastEthernet0/1 - 2

! 4. Rebuild cleanly (forcing DTP synchronization)

SW1(config)# interface range FastEthernet0/1 - 2

SW1(config-if-range)# switchport mode dynamic desirable

SW1(config-if-range)# channel-group 1 mode active

- ⚡ Section 8: Quick Memory Cheat Sheet for Exam Day

- LACP Keywords: active / passive (Standard)

- PAgP Keywords: desirable / auto (Cisco Only)

- Static Keywords: on (Dangerous)

- Optimal Load Balancing: port-channel load-balance src-dst-ip

- Good State: (SU) and (P)

- Bad State: (SD) and (s) (Suspended)

- The Golden Rule: All physical ports in a bundle must be identical clones (Speed, Duplex, VLANs, Trunk Mode). Configure the interface port-channel, NOT the physical interfaces!