Converting a switch to VTP SERVER

Switch> enable

Switch# configure terminal

Switch(config)# hostname SW3

SW3(config)# vtp domain CCNA-LAB

SW3(config)# vtp mode server

SW3(config)# exit

SW3# show vtp status

What each line does:

- vtp domain CCNA-LAB — names the VTP domain (must match on all switches)

- vtp mode server — this switch becomes the master VLAN database holder

- show vtp status — verify mode says "Server" and domain is correct

Now create VLANs on the server (this is where VLANs are made)

SW3(config)# vlan 10

SW3(config-vlan)# name Sales

SW3(config-vlan)# exit

SW3(config)# vlan 20

SW3(config-vlan)# name IT

SW3(config-vlan)# exit

Each vlan X command increases the revision number by 1 — this is what propagates to clients.

Converting a switch to VTP CLIENT

Switch> enable

Switch# configure terminal

Switch(config)# hostname SW1

SW1(config)# vtp domain CCNA-LAB

SW1(config)# vtp mode client

SW1(config)# exit

SW1# show vtp status

What each line does:

- vtp domain CCNA-LAB — MUST match the server's domain exactly (case-sensitive)

- vtp mode client — this switch will RECEIVE the VLAN database, cannot create its own VLANs

- show vtp status — verify mode says "Client"

The critical requirement — trunk must exist FIRST

VTP advertisements only travel over trunk links. So before any of this works:

SW3(config)# interface fastethernet 0/3

SW3(config-if)# switchport mode trunk

SW3(config-if)# exit

SW1(config)# interface fastethernet 0/4

SW1(config-if)# switchport mode trunk

SW1(config-if)# exit

(Use whichever ports actually connect SW3 to SW1 in your topology.)

Verification — the proof it worked

SW1# show vlan brief

If you see VLAN 10 (Sales) and VLAN 20 (IT) listed on SW1 — without ever typing vlan 10 on SW1 — VTP worked correctly.

Order of operations summary

1. Configure trunk ports FIRST (on both switches)

2. Set hostname + vtp domain + vtp mode server  (on the server switch)

3. Create VLANs on the server

4. Set hostname + vtp domain + vtp mode client  (on client switches)

5. Wait a few seconds

6. show vlan brief on clients → VLANs should appear automatically

Cable types — corrected rule for Packet Tracer

PC ↔ Switch     → Straight-through

Switch ↔ Switch → Crossover

Switch ↔ Router → Straight-through

Router ↔ Router → Crossover

Rule: same device type = crossover, different = straight-through

(Real hardware has Auto-MDIX and doesn't care — Packet Tracer does)

Port status meanings

connected            → cable + config OK, link up

notconnect           → no cable detected on this end

disabled             → port administratively shut down (needs "no shutdown")

down, line protocol down (disabled) → err-disabled, config conflict

VTP — VLAN Trunking Protocol

Server      → can create/edit VLANs, advertises to others

Client      → receives VLAN DB from server, can't create own VLANs

Transparent → ignores VTP, keeps local DB, but forwards advertisements

Requirements to sync:

1. Same VTP domain name (case-sensitive)

2. Connected via trunk link

3. Client revision number < server revision number

Native VLAN

One VLAN per trunk sent UNTAGGED (default = VLAN 1)

Best practice: change to unused VLAN (e.g. 99), must MATCH on both ends

Mismatch → link stays UP, but traffic leaks between VLANs (silent failure)

Commands used today

vtp domain CCNA-LAB              → set VTP domain (must match on all switches)

vtp mode server / client         → set VTP role

switchport trunk native vlan 99  → set native VLAN on trunk

interface range fa0/3 - 4        → configure multiple interfaces at once

show vtp status                  → check domain, mode, revision number

show interfaces fa0/x status     → check port connection status

show interfaces trunk            → check trunk mode, native vlan, allowed vlans

The core problem it solves

802.1Q tagging adds 4 extra bytes to every frame on a trunk. But this standard was created in 1998 — before that, switches and devices had no idea what a "tag" was.

If EVERY frame on a trunk got tagged, including the

oldest/simplest devices, those old devices would receive

a frame with an extra 4-byte header they don't understand

and might drop it or error out.

The solution — native VLAN

One VLAN on the trunk is designated "native"

Frames belonging to THAT VLAN are sent WITHOUT a tag

(exactly like a normal pre-802.1Q frame)

All other VLANs get tagged normally

This way, if an old device or a simple hub somehow ends up on a trunk link, untagged traffic (native VLAN) still works fine for it — while modern switches handle the tagged VLANs correctly.

Where VTP is configured

VTP is configured on each switch individually, in global config mode:

SW3(config)# vtp domain CCNA-LAB

SW3(config)# vtp mode server

That's it — two lines per switch (domain name + mode). You set this once per switch.

How it actually works — the simple version

Think of it like a shared notebook of VLAN names.

SW3 = the switch holding the "master notebook" (Server)

SW1, SW2 = switches that just copy whatever's in the notebook (Clients)

When you write a new VLAN into SW3's notebook (vlan 10 / name Sales), SW3 automatically photocopies that page and sends it to SW1 and SW2 over the trunk links. SW1 and SW2 update their own notebooks to match — without anyone typing vlan 10 on them directly.

The actual mechanism

1. You create VLAN 10 on SW3 (the server)

2. SW3's "revision number" goes up by 1

3. SW3 sends a VTP advertisement out all trunk ports

4. SW1/SW2 (clients) receive it

5. They compare: "is this revision number HIGHER than mine?"

6. Yes → they update their VLAN database to match SW3

Why it's used in real networks

Imagine a building with 20 switches. Without VTP, you'd manually type vlan 10 / name Sales on all 20 switches every time you add a department. With VTP, you type it once on the server, and all 20 update automatically.