- Rapid PVST+ (single block config)

conf t
spanning-tree mode rapid-pvst
exit

- 🟡 PortFast + BPDU Guard (combined per interface range)

conf t
interface range fa0/1 - 2
spanning-tree portfast
spanning-tree bpduguard enable
exit

- Rapid PVST+

Definition

Rapid PVST+ is Cisco’s implementation of Rapid Spanning Tree Protocol (802.1w) on a per-VLAN basis.

Purpose

- Prevent Layer 2 loops

- Provide fast recovery after link failure

- Improve network convergence time per VLAN

Why it is better than STP (802.1D / PVST)

- Convergence is much faster (seconds instead of 30–50 seconds)

- Uses event-driven logic instead of timers

- Reduces network downtime during topology changes

Key point

Rapid PVST+ continuously recalculates topology quickly instead of waiting for STP timers.

- PortFast

Definition

PortFast is a feature that makes a switch port immediately transition to the forwarding state by skipping STP listening and learning states.

Purpose

- Provide instant connectivity for end devices

- Eliminate STP delay during device boot or link-up

When to use

- PCs

- Laptops

- Printers

- Servers

Important restriction

Do not use on switch-to-switch links or trunk ports.

- BPDU Guard

Definition

BPDU Guard disables a PortFast-enabled port if it receives a BPDU frame.

Purpose

- Prevent accidental switching loops

- Protect access ports from rogue switches or hubs

What happens when a switch/hub is connected

- The device sends BPDUs

- BPDU Guard detects them

- The port is placed in err-disabled state

- The port stops forwarding traffic completely

- Recovery of BPDU Guard port

Manual recovery

- Shutdown interface

- No shutdown interface

Example

- Interface is err-disabled due to BPDU Guard

- Reset restores normal operation

- Key summary

- Rapid PVST+ = fast loop prevention (network-wide STP improvement)

- PortFast = immediate forwarding for end devices

- BPDU Guard = protection mechanism that shuts ports on BPDU detection