conf t

interface range fa0/3 - 4
channel-group 1 mode active
exit

interface port-channel 1
switchport mode trunk
end

EtherChannel is a Layer 2 technology that aggregates multiple physical links into a single logical link called a Port-Channel. LACP (Link Aggregation Control Protocol, IEEE 802.3ad/802.1AX) is the standard protocol used to negotiate and manage this aggregation.

Purpose

- Increase bandwidth by combining multiple links

- Provide redundancy and fast failover

- Prevent STP from blocking redundant links

- Improve load balancing across active links

LACP Operation

LACP dynamically forms EtherChannel by exchanging control packets between devices.

Modes

- Active: actively initiates LACP negotiation

- Passive: waits for LACP requests

- Requirement: at least one side must be active for the channel to form

Configuration Concept

- Physical interfaces are added to a channel group

- A logical interface (Port-Channel) is created

- STP treats Port-Channel as a single link

Key requirement:
All member interfaces must match:

- Speed

- Duplex

- VLAN / trunk configuration

Verification Commands

- show etherchannel summary → confirms bundle status (SU, P indicators)

- show interfaces port-channel 1 → logical interface details

- show spanning-tree → Port-Channel appears as one STP link

STP Behavior Impact

- Without EtherChannel: STP blocks redundant links

- With EtherChannel: all links are active; STP sees only one logical interface

Failure Handling

- If one physical link fails, traffic continues over remaining links

- No STP reconvergence required

- No traffic interruption during failover

Key Summary

EtherChannel with LACP combines multiple physical links into one logical link, increasing bandwidth, providing redundancy, and eliminating STP blocking between parallel links through automatic negotiation and load balancing.