1. Core Concept: Why Routing Exists

A router exists to connect different IP networks (subnets). A switch cannot do this because:

- A switch operates at Layer 2 (Data Link) → forwards frames using MAC addresses

- A router operates at Layer 3 (Network) → forwards packets using IP addresses

A switch has no understanding of IP networks, only device-to-device communication within the same subnet.
A router’s entire function is to interpret destination IP addresses and choose the correct network path.

- 2. How a Router Decides Where to Send Traffic

When a router receives a packet:

- Removes Layer 2 header (Ethernet frame)

- Reads destination IP address

- Compares it against its routing table

- Chooses:

  - A directly connected route (C)

  - Or a static route (S)

- Re-encapsulates packet into a new frame and forwards it

- 3. Routing Table Types

🔵 1. Connected Routes (C)

Automatically created when:

- Interface has an IP address

- Interface is enabled (no shutdown)

Command to verify:

show ip route

Example:

C 192.168.1.0/24 is directly connected, GigabitEthernet0/0

🟠 2. Static Routes (S)

Manually configured routes.

Command:

ip route <destination-network> <subnet-mask> <next-hop-ip>

Example:

ip route 10.0.0.0 255.255.255.0 172.16.0.2

Meaning:

“To reach 10.0.0.0/24, send packets to 172.16.0.2”

- 4. PC Routing Logic (Default Gateway)

A PC decides forwarding like this:

- If destination is in same subnet → send directly (ARP)

- If destination is in different subnet → send to default gateway

Default gateway = router interface IP

Without correct gateway:

- ❌ Can communicate only within local subnet

- ❌ Cannot reach other networks

- 5. IP vs MAC Behavior During Routing

| Field | Changes? | Why |
| --- | --- | --- |
| Source IP | ❌ No | End-to-end identity preserved |
| Destination IP | ❌ No | End-to-end delivery target |
| Source MAC | ✅ Yes | Changes at every hop |
| Destination MAC | ✅ Yes | Rewritten per next hop |

👉 Key idea:
IP packet stays constant, Ethernet frame is rebuilt at every router.

- 6. Static Routing Requirement Logic

If a router knows only directly connected networks:

- It cannot reach remote networks unless told

So:

- R1 must know how to reach R2’s LAN

- R2 must know how to reach R1’s LAN

Otherwise:

- Forward path may work

- Return path fails (asymmetric routing issue)

- 7. Packet Flow Example (End-to-End)

PC-A → R1 → R2 → PC-C

- PC-A sends packet to gateway (R1)

- R1 checks routing table → sends to R2 via static route

- R2 checks routing table → sends to PC-C network

- PC-C replies using its gateway (R2)

- Reverse path must also exist

- 🛠️ Essential Configuration Commands

- Router Interface Setup

conf t
interface g0/0
 ip address 192.168.1.1 255.255.255.0
 no shutdown

- Static Route

ip route 192.168.3.0 255.255.255.0 192.168.100.2

- Verify Routing Table

show ip route

- Check Interfaces

show ip interface brief

- Test Connectivity

ping 192.168.3.10
traceroute 192.168.3.10

- 📌 When to Use What

| Tool | When to Use |
| --- | --- |
| Connected route (C) | Directly attached networks |
| Static route (S) | Small networks, manual control, labs |
| Default gateway | Always on end devices |
| Router | Any time subnets must communicate |
| Switch | Same subnet communication only |

- 🧠 End-of-Day Questions (Answered)

- 1. Why can a switch never connect two subnets?

Because a switch operates at Layer 2, forwarding based on MAC addresses only. It does not inspect or route based on IP networks, so it cannot interpret or separate subnets.

- 2. Conditions for a “C” route

A connected route appears when:

- Interface has a valid IP address

- Interface is administratively up (no shutdown)

- 3. Static route command

ip route 10.0.0.0 255.255.255.0 172.16.0.2

- 4. Wrong default gateway

- Same subnet ping → ✅ works

- Different subnet ping → ❌ fails

Because routing to external networks depends entirely on gateway.

- 5. Do IP/MAC addresses change?

- IP addresses → ❌ never change

- MAC addresses → ✅ change at every router hop

- 6. Missing return route

Ping will:

- ❌ reach destination

- ❌ fail on reply (no return path)

This creates one-way connectivity failure

- 7. PC vs Router decision process

- PC: decides based on same subnet vs default gateway

- Router: decides using routing table lookup

- 8. Unreachable next-hop in static route

The route becomes invalid for forwarding, so packets:

- ❌ are dropped

- or never forwarded correctly