Core Concepts

- DORA Process: The fundamental four-step handshake for dynamic address assignment.

  - Discover: Client broadcasts a request for a DHCP server.

  - Offer: Server proposes an available IP address.

  - Request: Client formally requests to lease that specific IP.

  - Acknowledge: Server confirms lease details and adds to the binding table.

- Excluded Addresses: Critical to reserve IP ranges (like gateways or static servers) to prevent IP conflicts.

- Pool Management: Defining the scope of addresses, the gateway, and optional parameters (DNS, domain names, lease duration).

Configuration Workflow

1. Preparation & Exclusions

Before defining the pool, remove static infrastructure IPs from the assignable range.

Plaintext

! Enable the DHCP service (if not already running)

Router(config)# service dhcp

! Exclude the Gateway and the first 9 addresses to avoid IP conflicts

Router(config)# ip dhcp excluded-address 192.168.10.1 192.168.10.10

2. Defining the DHCP Pool

This is where the network parameters are configured for the client devices.

Plaintext

! Create the pool and enter configuration mode

Router(config)# ip dhcp pool VLAN10_POOL

! Set the network subnet and mask

Router(dhcp-config)# network 192.168.10.0 255.255.255.0

! Set the default gateway for the clients

Router(dhcp-config)# default-router 192.168.10.1

! Provide DNS servers

Router(dhcp-config)# dns-server 8.8.8.8 1.1.1.1

! Set lease duration (in days)

Router(dhcp-config)# lease 1

Verification & Troubleshooting

- Binding Table: Use this to confirm the lease and the client's identity (MAC address).

Plaintext

Router# show ip dhcp binding

- Pool Statistics: Use this to see how many addresses remain in the pool and general usage.

Plaintext

Router# show ip dhcp pool

- Troubleshooting: If the DORA process fails, verify connectivity, ensure the service dhcp is enabled, and check that there are no overlapping network scopes.