1. Inter-VLAN Routing (Router-on-a-Stick)

Inter-VLAN routing enables communication between different VLANs by using a single physical

router interface acting as a gateway for multiple VLANs.

● Concept: A "Trunk" link carries traffic for multiple VLANs between the switch and the

router. The router uses subinterfaces to handle each VLAN separately.

● Key Commands:

! Define the subinterface

interface GigabitEthernet0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

● Why it works: The encapsulation dot1Q [VLAN_ID] command tells the router to tag

outgoing packets and strip/recognize tags from incoming packets for that specific VLAN.

2. Access Control Lists (ACLs)

ACLs act as filters to permit or deny traffic based on IP addresses, ports, or protocols.

● Directional Logic: ACLs are applied to an interface in a specific direction:

○ Inbound (in): Filters traffic entering the router interface (before routing).

○ Outbound (out): Filters traffic leaving the router interface (after routing).

● Stateless Nature: Standard extended ACLs are stateless. They treat every packet as an

individual event and do not "remember" sessions. This causes issues with return traffic

(e.g., if you block VLAN 20 from talking to VLAN 10, the reply from VLAN 10 back to

VLAN 20 is also blocked).

3. Resolving the "One-Way" Communication

Challenge

To allow VLAN 10 to communicate with VLAN 20, but prevent VLAN 20 from initiating traffic to

VLAN 10, you must account for the stateless nature of the ACL.

● Recommended Configuration:

ip access-list extended BLOCK_GUEST

! Explicitly permit Echo Replies so return traffic works

permit icmp 192.168.10.0 0.0.0.255 192.168.20.0 0.0.0.255

echo-reply

! Deny the initiation of traffic from Guest to Confidential

deny ip 192.168.20.0 0.0.0.255 192.168.10.0 0.0.0.255

! Permit all other traffic (e.g., Internet)

permit ip any any

● Implementation: Apply this to the Guest subinterface:

interface gig0/0.20

ip access-group BLOCK_GUEST in

4. Troubleshooting Summary

● show ip interface brief: Verify subinterfaces are up/up.

● show vlan brief: Ensure switch ports are assigned to the correct VLANs.

● show ip access-list: Check the "matches" counter to confirm the ACL is catching traffic.

● Check End-Device Gateways: Ensure PCs have the router subinterface IP set as their

Default Gateway.