🔷 VLAN

- Logical network separation (department-wise)

- Each VLAN = separate broadcast domain

- Same VLAN = can communicate, different VLAN = cannot

🔷 VLANs used

- VLAN 10 = Operations

- VLAN 20 = Warehouse

- VLAN 30 = Admin

- VLAN 99 = Management (native VLAN)

🔷 ACCESS PORT

- Connects PC to switch

switchport mode access
switchport access vlan X

🔷 TRUNK PORT

- Switch ↔ Switch link (carries multiple VLANs)

switchport mode trunk
switchport trunk native vlan 99

🔷 ETHERCHANNEL

- Bundles multiple links into 1 logical link

channel-group 1 mode active

- Faster + redundancy + STP sees 1 link

🔷 STP (Spanning Tree)

- Prevents loops by blocking redundant paths

- One path active, others blocked

🔷 RAPID PVST

spanning-tree mode rapid-pvst

- Fast STP

- One STP per VLAN

🔷 ROOT BRIDGE (CORE RULE)

- Core switch = root for all VLANs

spanning-tree vlan 10,20,30,99 priority 4096

🔷 VTP

- Auto VLAN sharing between switches

- Core = server, others = client

🔷 PING RULE

- Same VLAN → ✔ success

- Different VLAN → ❌ fail

🔷 KEY FLOW

PC → Access Port → Switch → EtherChannel → Trunk → Core → VLAN logic