Single-area OSPF is great for small networks, but as networks grow, it causes massive CPU and memory overhead because every router must maintain an identical, full-scale map of every link. Multi-Area OSPF introduces hierarchy and scalability by dividing the network into functional zones.

- 1. The Multi-Area Architecture

OSPF divides the network into a Backbone Area and Non-Backbone Areas.

- Area 0 (Backbone): The mandatory core of the OSPF network. All other areas must connect directly to Area 0 to pass traffic to one another.

- Non-Backbone Areas (Area 1, 2, etc.): Localized zones that maintain their own internal topology.

- The Golden Rule: All inter-area traffic must transit through Area 0.

- 2. Key Terminology

- ABR (Area Border Router): A router that has at least one interface in Area 0 and at least one interface in a non-backbone area. The ABR acts as the "translator" or "gateway" that summarizes information between areas.

- Intra-Area Routing: Traffic that stays within the same area. Routers have a full, detailed map of the topology.

- Inter-Area Routing: Traffic that crosses area boundaries. Routers only see "Summary" routes (O IA) provided by the ABR, not the granular details of the remote area.

- 3. Why Use Multiple Areas? (The Practical Benefits)

- Blast Radius Reduction: If a link flaps in Area 1, only the routers inside Area 1 are forced to run the SPF (Shortest Path First) algorithm. Area 0 and other areas remain stable.

- Hardware Efficiency: Branch routers do not need to store the massive link-state database of the entire corporation. They only need to know their local area and a "default/summary" path to the rest of the world.

- LSA Flooding Control: Link-State Advertisements (LSAs) are contained within their specific area. The ABR prevents "chatter" from flooding the entire enterprise.

- 4. Verification & Diagnostics

When you configure multiple areas, the routing table and database change significantly.

Identifying Inter-Area Routes

When you run show ip route ospf, look for the O IA code:

- O: Standard OSPF (Intra-area).

- O IA: OSPF Inter-Area (routes learned from a different area via an ABR).

Verifying ABR Status

On an ABR, the command show ip ospf will explicitly state its role:

Plaintext

Router# show ip ospf

...

It is an Area Border Router

...

Checking the Database

Running show ip ospf database will now show:

- Router Link States: Detailed maps of your local area.

- Summary Link States: The "briefs" provided by the ABR about networks in other areas.

- 5. Day 23 Command Cheat Sheet

| Task | Command Syntax |
| --- | --- |
| Assign network to area | Router(config-router)# network <IP> <Wildcard> area <ID> |
| View routing table | Router# show ip route ospf |
| View OSPF process info | Router# show ip ospf |
| View LSDB | Router# show ip ospf database |