1. OSPF Adjacency Troubleshooting (The "Friendship" Phase)

When routers fail to reach the FULL state, they are stuck in a specific handshake step:

- Init State: Routers are hearing each other, but the handshake is incomplete.

  - Cause: Mismatched Hello/Dead timers or incorrect Authentication.

- ExStart/Exchange State: Routers are trying to exchange their "map" (database).

  - Cause: MTU Mismatch (packets are too large) or Duplicate Router IDs.

- Loading State: Routers are requesting specific missing data.

  - Cause: Connectivity issues or interface flapping.

2. The Golden Rules of OSPF Adjacency

For two routers to form an adjacency, ALL of the following must match perfectly:

- Subnet/Mask: They must be in the same network segment.

- Area ID: They must be in the same OSPF Area.

- Timers: Hello and Dead intervals must be identical.

- Authentication: If used, the password/key must match.

- Stub Flags: (For advanced setups) Stub area flags must align.

- MTU: Must match (or be ignored) for the database to sync successfully.

3. Missing Routes (The "Data" Phase)

If your neighbors are FULL but routes are missing from your routing table:

- Network Command: Check if the interface is actually included in the router ospf process.

- Passive Interface: Ensure you haven't enabled passive-interface on an active link.

- Database Check: Use show ip ospf database to see if the LSA exists. If it's missing from the database, the router has never "heard" about that network.

4. Essential Troubleshooting Toolkit

- show ip ospf neighbor: The #1 command. Look at the State column.

- show ip ospf interface <int>: Use this to verify timers, costs, and Area assignments.

- show ip ospf database: Use this to see the "map" the router has built.

- debug ip ospf adj: The "live" view of the handshake. Use carefully!

- clear ip ospf process: The "factory reset" for the OSPF protocol. Use it to force a renegotiation after making configuration changes.

5. Pro-Tips for the CLI

- The "do" command: Use do (e.g., do show ip route) when you are inside configuration mode (config-if)# so you don't have to exit.

- The "?" key: When stuck, type ? after any command to see available options.

- Ambiguity: If you get an "Ambiguous command" error, type more letters (e.g., show ip ospf neigh becomes show ip ospf neighbor).