Here are your consolidated study notes for Day 52. You can copy and paste this directly into your master study document!

- Lab Notes: Day 52 - SSH & Device Management

1. The Core Concept: Telnet vs. SSH

- Telnet is dead: Telnet transmits all data—including usernames, passwords, and configuration commands—in plain cleartext. Anyone running a packet sniffer can steal your credentials.

- SSH (Secure Shell) is the standard: SSH uses strong cryptographic algorithms to build an encrypted tunnel. If intercepted, the packets look like mathematical gibberish.

- The Management Plane: Configuring SSH secures the Management Plane—the private, logical backchannel used exclusively by IT administrators to control the device (separate from the Data and Control planes).

2. The SSH Configuration Sequence

Cisco requires a strict order of operations to enable SSH on a router or switch. If you skip a step, the commands will fail.

Plaintext

! 1. Set the hostname (required for crypto keys)

Router(config)# hostname R1

! 2. Define the domain name (required for crypto keys)

R1(config)# ip domain-name cisco.com

! 3. Generate the RSA encryption keys (1024 or 2048-bit)

R1(config)# crypto key generate rsa

How many bits in the modulus [512]: 2048

! 4. Enforce SSH version 2 (security best practice)

R1(config)# ip ssh version 2

! 5. Create a local username and encrypted password

R1(config)# username admin secret cisco123

! 6. Lock down the VTY (Virtual Teletype) lines

R1(config)# line vty 0 4             <-- (Use 0 15 for switches)

R1(config-line)# login local         <-- (Forces the router to use the local user database)

R1(config-line)# transport input ssh <-- (Kills Telnet, only allows SSH)

! 7. Set the Enable Secret (CRITICAL)

R1(config)# enable secret cisco      <-- (Without this, remote users cannot enter Privileged EXEC mode)

3. Testing & Troubleshooting

- The Connection Command: From a PC command prompt, use ssh -l <username> <ip_address>. Make sure to use a lowercase 'L', not a number 1.

  - Example: C:\> ssh -l admin 192.168.1.1

- The "% No password set" Error: If you log in via SSH but the switch/router rejects the enable command, it means you forgot to set the enable secret. Cisco strictly prohibits remote configuration if the master lock is not set on the device.

4. Architectural Context: Centralized Network Monitoring

Setting up SSH is the absolute baseline for enterprise infrastructure architecture. When building a centralized network monitoring platform, this encrypted access is what allows the dashboard to communicate with the hardware.

- Secure CRUD Operations: Rather than relying on automatic discovery, administrators use these secure VTY lines to manually register, read, update, and delete device configurations from a central dashboard.

- Role-Based Access: By creating multiple local usernames (admin vs. technician) and setting different privilege levels, the management plane can enforce strict user roles before anyone can access the troubleshooting guidance on the platform.