- OSPFv3 for IPv6 & Dual-Stack Routing

- 📌 Section 1: Core Mechanics (WHAT, WHY, WHEN, HOW)

🔹 What is OSPFv3?

- WHAT: OSPFv3 is the IPv6-capable version of the Open Shortest Path First (OSPF) link-state routing protocol (defined in RFC 5340).

- WHY: OSPFv2 was built strictly for IPv4 and cannot advertise IPv6 network prefixes. OSPFv3 was redesigned from the ground up to support IPv6 headers, link-local neighbor relationships, and multi-protocol capabilities.

- WHEN: Deployed when dynamic routing is needed across IPv6-only enterprise networks or Dual-Stack networks where IPv4 and IPv6 run side-by-side.

- HOW: It forms neighbor adjacencies using ICMPv6 Link-Local (FE80::) addresses, enables routing directly on individual interfaces, and uses IPv6 multicast addresses (FF02::5 and FF02::6).

- 🔑 Section 2: The 3 Golden Pillars of OSPFv3

1. Interface-Level Activation (Port-Level Config)

- OSPFv2 (IPv4): Configured globally under the process using network commands and wildcard masks:

Plaintext

R1(config-router)# network 10.0.12.0 0.0.0.255 area 0

- OSPFv3 (IPv6): No network commands exist! Activated directly on the specific interface:

Plaintext

R1(config-if)# ipv6 ospf <process-id> area <area-id>

2. Mandatory 32-Bit Router ID (RID)

- Even though OSPFv3 routes 128-bit IPv6 addresses, the Router ID is STILL a 32-bit integer formatted like an IPv4 address (e.g., 1.1.1.1 or 2.2.2.2).

- ⚠️ The Exam & Lab Trap: If an IPv6-only router has no active IPv4 addresses configured on any interface, OSPFv3 will fail to start with an error until you manually assign a Router ID using router-id X.X.X.X under the process!

3. Link-Local Adjacencies (FE80::)

- OSPFv3 hello packets, neighbor adjacencies, and routing table next-hops all use Link-Local Addresses (FE80::), NOT Global Unicast Addresses (GUAs).

- Why it matters: Even if you change, re-IP, or temporarily lose the Global Unicast Address (2001:DB8::) on an interface, the OSPFv3 neighbor relationship stays completely intact over the FE80:: link.

- 📊 Section 3: OSPFv2 vs. OSPFv3 Comparison Table

| Feature / Metric | 🟢 OSPFv2 (IPv4) | 🔵 OSPFv3 (IPv6) |
| --- | --- | --- |
| Routed Protocol | IPv4 | IPv6 |
| Enable Command | network command in router process | Directly on the interface (ipv6 ospf 1 area 0) |
| Router ID (RID) | 32-bit dotted-decimal (1.1.1.1) | 32-bit dotted-decimal (1.1.1.1) (Requires manual config if IPv4 is missing) |
| Neighbor Adjacencies | Interface IPv4 Address | Link-Local Address (FE80::) |
| All-Routers Multicast | 224.0.0.5 | FF02::5 |
| DR / BDR Multicast | 224.0.0.6 | FF02::6 |
| Routing Table Next-Hop | Neighbor's IPv4 Address | Neighbor's Link-Local Address (FE80::) |
| Authentication | Native in OSPF header | Handled by native IPv6 IPsec |

- 🌐 Section 4: Dual-Stack Routing Architecture

Dual-Stack allows IPv4 and IPv6 protocols to run simultaneously on the exact same physical routers and interface cables.

Plaintext

+-------------------------------------------------+

|              Physical Link (G0/0)               |

+-----------------------+-------------------------+

|

+-------------------+-------------------+

|                                       |

IPv4 Traffic (OSPFv2)                   IPv6 Traffic (OSPFv3)

Uses IP: 10.0.12.1                      Uses IP: 2001:DB8:12::1

Multicast: 224.0.0.5                    Multicast: FF02::5

- OSPFv2 Process: Handles IPv4 route discovery and populates the IPv4 routing table (show ip route).

- OSPFv3 Process: Handles IPv6 route discovery and populates the IPv6 routing table (show ipv6 route).

- Both processes maintain completely separate neighbor tables, database tables, and routing tables.

- 🗺️ Section 5: Lab Topology & Addressing Scheme

Plaintext

[ R1 ] <================ WAN Link 1 =================> [ R2 ] <================ WAN Link 2 =================> [ R3 ]

(Router 1)               2001:db8:12::/64              (Router 2)               2001:db8:23::/64              (Router 3)

RID: 1.1.1.1               10.0.12.0/24                 RID: 2.2.2.2               10.0.23.0/24                 RID: 3.3.3.3

Lo0: 2001:db8:1::1/64    (G0/0)              (G0/0)     Lo0: 2001:db8:2::1/64    (G0/1)              (G0/0)     Lo0: 2001:db8:3::1/64

Lo0: 1.1.1.1/32                                         Lo0: 2.2.2.2/32                                         Lo0: 3.3.3.3/32

<------------------------------------------------- ALL INTERFACES IN AREA 0 ------------------------------------------------->

Addressing Table

| Device | Interface | IPv4 Address | IPv6 GUA Address | IPv6 Link-Local | OSPF Area |
| --- | --- | --- | --- | --- | --- |
| R1 | Loopback0 | 1.1.1.1/32 | 2001:DB8:1::1/64 | N/A | Area 0 |
|  | GigabitEthernet0/0 | 10.0.12.1/24 | 2001:DB8:12::1/64 | FE80::1 | Area 0 |
| R2 | Loopback0 | 2.2.2.2/32 | 2001:DB8:2::1/64 | N/A | Area 0 |
|  | GigabitEthernet0/0 | 10.0.12.2/24 | 2001:DB8:12::2/64 | FE80::2 | Area 0 |
|  | GigabitEthernet0/1 | 10.0.23.2/24 | 2001:DB8:23::2/64 | FE80::2 | Area 0 |
| R3 | Loopback0 | 3.3.3.3/32 | 2001:DB8:3::1/64 | N/A | Area 0 |
|  | GigabitEthernet0/0 | 10.0.23.3/24 | 2001:DB8:23::3/64 | FE80::3 | Area 0 |

- 📜 Section 6: Complete CLI Configuration Scripts

🏢 Router 1 (R1)

Plaintext

enable

configure terminal

hostname R1

! --- Enable IPv6 Forwarding ---

ipv6 unicast-routing

! --- Enable Routing Processes ---

router ospf 1

router-id 1.1.1.1

exit

ipv6 router ospf 1

router-id 1.1.1.1

exit

! --- Interfaces ---

interface Loopback0

description Management Loopback

ip address 1.1.1.1 255.255.255.255

ip ospf 1 area 0

ipv6 address 2001:DB8:1::1/64

ipv6 ospf 1 area 0

exit

interface GigabitEthernet0/0

description WAN to R2

ip address 10.0.12.1 255.255.255.0

ip ospf 1 area 0

ipv6 address 2001:DB8:12::1/64

ipv6 address FE80::1 link-local

ipv6 ospf 1 area 0

no shutdown

exit

end

write memory

🏢 Router 2 (R2 — Core Router)

Plaintext

enable

configure terminal

hostname R2

! --- Enable IPv6 Forwarding ---

ipv6 unicast-routing

! --- Enable Routing Processes ---

router ospf 1

router-id 2.2.2.2

exit

ipv6 router ospf 1

router-id 2.2.2.2

exit

! --- Interfaces ---

interface Loopback0

description Management Loopback

ip address 2.2.2.2 255.255.255.255

ip ospf 1 area 0

ipv6 address 2001:DB8:2::1/64

ipv6 ospf 1 area 0

exit

interface GigabitEthernet0/0

description WAN to R1

ip address 10.0.12.2 255.255.255.0

ip ospf 1 area 0

ipv6 address 2001:DB8:12::2/64

ipv6 address FE80::2 link-local

ipv6 ospf 1 area 0

no shutdown

exit

interface GigabitEthernet0/1

description WAN to R3

ip address 10.0.23.2 255.255.255.0

ip ospf 1 area 0

ipv6 address 2001:DB8:23::2/64

ipv6 address FE80::2 link-local

ipv6 ospf 1 area 0

no shutdown

exit

end

write memory

🏢 Router 3 (R3)

Plaintext

enable

configure terminal

hostname R3

! --- Enable IPv6 Forwarding ---

ipv6 unicast-routing

! --- Enable Routing Processes ---

router ospf 1

router-id 3.3.3.3

exit

ipv6 router ospf 1

router-id 3.3.3.3

exit

! --- Interfaces ---

interface Loopback0

description Management Loopback

ip address 3.3.3.3 255.255.255.255

ip ospf 1 area 0

ipv6 address 2001:DB8:3::1/64

ipv6 ospf 1 area 0

exit

interface GigabitEthernet0/0

description WAN to R2

ip address 10.0.23.3 255.255.255.0

ip ospf 1 area 0

ipv6 address 2001:DB8:23::3/64

ipv6 address FE80::3 link-local

ipv6 ospf 1 area 0

no shutdown

exit

end

write memory

- 🔬 Section 7: Real Lab Verification & Critical Routing Table Insights

1. Verifying Neighbor Table

Plaintext

R2# show ipv6 ospf neighbor

Plaintext

Neighbor ID     Pri   State           Dead Time   Interface ID    Interface

1.1.1.1           1   FULL/BDR        00:00:33    1               GigabitEthernet0/0

3.3.3.3           1   FULL/DR         00:00:32    1               GigabitEthernet0/1

2. Diagnostic Deep Dive: C vs O vs L Route Types

In today's lab, running show ipv6 route on R2 yielded 9 entries:

Plaintext

O   2001:DB8:1::1/128 [110/1] via FE80::1, GigabitEthernet0/0

C   2001:DB8:2::/64 [0/0] via Loopback0, directly connected

L   2001:DB8:2::1/128 [0/0] via Loopback0, receive

O   2001:DB8:3::1/128 [110/1] via FE80::3, GigabitEthernet0/1

C   2001:DB8:12::/64 [0/0] via GigabitEthernet0/0, directly connected

L   2001:DB8:12::2/128 [0/0] via GigabitEthernet0/0, receive

C   2001:DB8:23::/64 [0/0] via GigabitEthernet0/1, directly connected

L   2001:DB8:23::2/128 [0/0] via GigabitEthernet0/1, receive

Why doesn't 2001:DB8:23::/64 show up as an O (OSPF) route on R2?

- Connected (C) Routes have an Administrative Distance (AD) of 0.

- Because 2001:DB8:23::/64 is physically plugged into R2's Gig0/1 port, R2 already knows about it directly (AD = 0). It will never override a directly connected link with an OSPF route (AD = 110).

- Where DOES it show up as O? On R1! For R1, 2001:DB8:23::/64 is a remote network learned across R2.

Understanding Route Codes:

- C (Connected): Entire /64 subnet plugged directly into a physical or virtual interface.

- L (Local Host Route): Specific /128 IP assigned to the router's own interface. Tells the CPU to process incoming packets locally instead of forwarding them.

- O (OSPF Intra-Area): Remote networks learned dynamically from neighboring routers via OSPFv3.

- ⚡ Section 8: Cheat Sheet for CCNA Exam Day

- OSPFv2 Enable Method: Global config process → network <ip> <wildcard> area <area>

- OSPFv3 Enable Method: Interface config → ipv6 ospf <process-id> area <area-id>

- OSPFv3 Router ID: 32-bit dotted-decimal string (X.X.X.X). Required if no IPv4 addresses are present.

- OSPFv3 Transport & Next-Hop: Always uses Link-Local (FE80::) addresses.

- Check IPv6 OSPF Neighbors: show ipv6 ospf neighbor

- Check Interface OSPF Info: show ipv6 ospf interface brief

- Check IPv6 Routing Table: show ipv6 route (or show ipv6 route ospf)