Day 57 Troubleshooting & Concept Notes

1. Native VLAN Mismatches (The Silent Killer)

- What it is: When the Trunk link between a switch and a router (or switch and switch) has different Native VLANs configured on each end.

- The Symptom: Packet Tracer flashes CDP mismatch errors (%CDP-4-NATIVE_VLAN_MISMATCH), and untagged traffic fails to cross the link.

- The Fix: Ensure both sides match.

  - Switch Side (SW_CORE):

Code snippet

interface fa0/24

switchport trunk native vlan 999

  - Router Side (R_CORE sub-interface):

Code snippet

interface gig0/0.999

encapsulation dot1Q 999 native

2. Ping Failures from a Switch

- The Problem: Trying to ping a router (e.g., ping 172.16.1.1) directly from a Layer 2 switch CLI fails, even if the Trunk is perfectly configured.

- The Reason: Layer 2 switches do not have IP addresses by default. When the switch sends the ping, the router doesn't know where to send the reply because there is no source IP.

- The Fix: Assign a Management IP (SVI - Switch Virtual Interface) to the switch in an active VLAN.

Code snippet

interface vlan 100

ip address 172.16.1.2 255.255.255.0

no shutdown

Note: It is generally better to test connectivity from an end-device (PC) rather than the switch itself.

3. Router-on-a-Stick vs. Router-to-Router Links

- Switch-to-Router (Router-on-a-Stick):

  - Uses a single physical cable sliced into virtual sub-interfaces (Gig0/0.100).

  - Requires the encapsulation dot1Q [vlan_id] command on the router so it understands the VLAN tags sent by the switch.

  - The physical interface (Gig0/0) gets NO IP address.

  - Drawback: Creates a massive bottleneck ("Hairpin turn") because all Inter-VLAN traffic travels up and down the exact same physical wire.

- Router-to-Router (e.g., R_CORE to R_EDGE):

  - Purely Layer 3.

  - NO VLAN tags, no sub-interfaces, no 802.1Q encapsulation.

  - IP addresses are assigned directly to the physical interface.

4. Troubleshooting Workflow (When the final ping fails)

If PC-VLAN100 cannot ping the external 8.8.4.4, trace the path logically:

- Gateway Check: Can the PC ping its Default Gateway (172.16.1.1)? (Tests the Trunk and Sub-interfaces).

- Edge Check: Can the PC ping the Edge Router's inside interface (10.10.10.1)? (Tests OSPF routing).

- NAT Check: Run show ip nat translations on the Edge Router while pinging. Is the traffic being translated? (Tests Access Lists and NAT overload).

- Route Check: Run show ip route on the Core Router. Does it have a default route (O*E2 0.0.0.0/0) injected via OSPF? (Tests default-information originate).