- DHCP

- Objective

Automatically assign IP addresses to clients without manual configuration.

- Real-world Requirement

"Employees should receive IP addresses automatically when they connect."

Technology Required

- DHCP Server

- Step 1 — Reserve Static Addresses

Command

ip dhcp excluded-address 192.168.10.1 192.168.10.5

What does it do?

Excludes addresses from DHCP assignment.

Why is it needed?

The router should never lease:

- Router Gateway

- Switch Management

- Servers

- Printers

These devices normally use static IPs.

Packet Flow

PC

│

│ DHCP Discover

▼

Router checks exclusion list

│

├── .1 Reserved

├── .2 Reserved

├── .3 Reserved

├── .4 Reserved

├── .5 Reserved

▼

Offers .6

Syntax

ip dhcp excluded-address START_IP END_IP

Single Address

ip dhcp excluded-address 192.168.10.1

Range

ip dhcp excluded-address 192.168.10.1 192.168.10.5

Verify

show running-config | section dhcp

Common Mistakes

❌ Forget exclusions

Result

Gateway receives dynamic address conflict.

Then

- Step 2 — Create DHCP Pool

Command

ip dhcp pool STAFF_POOL

What it does

Creates a DHCP scope.

Why needed

Without a pool, DHCP has nowhere to allocate addresses from.

Command

network 192.168.10.0 255.255.255.0

What it does

Defines the subnet.

Why

Router must know which network belongs to this pool.

Command

default-router 192.168.10.1

What it does

Sets the default gateway handed to clients.

Command

dns-server 8.8.8.8

What it does

Tells clients which DNS server to use.

Verification

show ip dhcp binding

show ip dhcp pool

show running-config

Troubleshooting

Client gets APIPA?

↓

DHCP Pool exists?

↓

Gateway excluded?

↓

Interface up?

↓

Correct VLAN?

↓

Correct helper?

↓

Pool exhausted?

Then

- NAT

Requirement

Internal users should access the Internet.

Technology

Dynamic PAT

Step 1

Define Inside

interface g0/0.10

ip nat inside

What

Marks interface as trusted.

Why

NAT only translates traffic crossing from inside to outside.

Step 2

Define Outside

interface g0/1

ip nat outside

Step 3

ACL

access-list 10 permit 192.168.10.0 0.0.0.255

Why Standard ACL?

Because NAT only cares about source addresses.

Step 4

Pool

ip nat pool PUBLIC_NAT ...

Explain

Start Address

End Address

Netmask

Pool Size

Step 5

PAT

ip nat inside source list 10 pool PUBLIC_NAT overload

Break down every word

inside

Traffic originates inside.

source

Translate source IP.

list 10

Match ACL 10.

pool

Use this public address range.

overload

PAT

Many devices

One/Few public IPs

Different TCP/UDP ports.

Verification

show ip nat translations

show ip nat statistics

Expected Output

Inside Local

↓

Inside Global

↓

Outside Global

Then

- ACL

Requirement

Block IoT from pinging Internet server.

Technology

Extended ACL

Rule

deny icmp 192.168.30.0 0.0.0.255 host 8.8.8.8 echo

Word-by-word explanation

deny

Drop packet.

icmp

Only ICMP.

Not TCP.

Not UDP.

192.168.30.0

Source Network.

0.0.0.255

Wildcard.

host

Exactly one destination.

echo

Ping Request only.

Then

Common mistakes

Putting ACL outbound instead of inbound.

Wrong interface.

Wrong order.

Implicit deny.

Wrong wildcard.

Then

Verification

show access-lists

show ip interface

Then

Packet Walk

PC

↓

DHCP

↓

Router

↓

ACL

↓

Routing

↓

NAT

↓

PAT

↓

WAN

↓

ISP

↓

Server

↓

Reply

↓

Reverse NAT

↓

PC

I would also add design sections that connect user requirements to the right technology. For example:

| User Requirement | Technology | Commands |
| --- | --- | --- |
| PCs should get IPs automatically | DHCP | ip dhcp pool, network, default-router |
| Separate HR and IoT devices | VLANs | vlan, switchport access vlan |
| Multiple VLANs over one cable | Trunking | switchport mode trunk, encapsulation dot1q |
| Allow VLANs to communicate | Router-on-a-Stick | interface g0/0.x, encapsulation dot1q |
| Internet access | Static Route + NAT/PAT | ip route, ip nat inside, overload |
| Share one public IP | PAT | overload |
| Block IoT → Staff | Extended ACL | deny ip ... |
| Allow only HTTP/HTTPS | Extended ACL | permit tcp ... eq 80, eq 443 |
| Prevent network loops | STP | spanning-tree |
| Increase bandwidth | EtherChannel | channel-group |
| Advertise routes automatically | OSPF/EIGRP | router ospf, network |

- …….****
🎯 Objective

Allow multiple internal devices (Staff and IoT) to access the Internet using a limited pool of public IP addresses.

- Business Requirement

The company has:

- Staff Network → 192.168.10.0/24

- IoT Network → 192.168.30.0/24

The ISP has assigned only these public IPs:

203.0.113.10

203.0.113.11

203.0.113.12

203.0.113.13

203.0.113.14

203.0.113.15

Every internal device must share these public addresses when accessing the Internet.

Technology Required:

- Static Route

- NAT

- PAT

- Standard ACL

- How PAT Works

Private Network

PC1      PC2       PC3

│         │         │

│         │         │

192.168.x.x

│

▼

Router

NAT + PAT Engine

│

▼

203.0.113.10:1001

203.0.113.10:1002

203.0.113.10:1003

│

▼

Internet

PAT keeps one translation table:

| Private IP | Public IP | Port |
| --- | --- | --- |
| 192.168.10.10 | 203.0.113.10 | 1025 |
| 192.168.10.20 | 203.0.113.10 | 1026 |
| 192.168.30.15 | 203.0.113.10 | 1027 |

Instead of using a unique public IP for every device, PAT differentiates connections by TCP/UDP port numbers.

- Step 1 – Configure a Static Default Route

Requirement

Configure a static default route pointing to the ISP gateway.

Command

ip route 0.0.0.0 0.0.0.0 192.168.12.2

Syntax

ip route

Destination

Subnet Mask

Next-Hop

Breakdown

0.0.0.0

Means

Every unknown destination.

0.0.0.0

Mask

Match every network.

192.168.12.2

Next-hop router.

Why Needed?

Without a default route

PC

↓

R1

↓

Destination = 8.8.8.8

R1

"I don't know where that network is."

Drop Packet.

With a default route

Unknown Network

↓

Forward packet

↓

ISP Router

Verify

show ip route

Expected

S* 0.0.0.0/0 via 192.168.12.2

- Step 2 – Define NAT Boundaries

Requirement

Which interfaces belong to:

Inside

Outside

- Inside

Internal LAN

interface GigabitEthernet0/0.10

ip nat inside

interface GigabitEthernet0/0.30

ip nat inside

What does this command do?

Marks the interface as trusted.

Traffic entering this interface is considered private.

- Outside

interface GigabitEthernet0/1

ip nat outside

Marks the ISP-facing interface.

Traffic leaving this interface will be translated.

Packet Flow

PC

↓

G0/0.10

(Inside)

↓

Routing

↓

NAT

↓

G0/1

(Outside)

↓

Internet

Verify

show running-config interface g0/0.10

show running-config interface g0/1

- Step 3 – Configure the NAT ACL

Requirement

Translate ONLY

192.168.10.0/24

192.168.30.0/24

Command

access-list 10 permit 192.168.10.0 0.0.0.255

access-list 10 permit 192.168.30.0 0.0.0.255

Why a Standard ACL?

NAT only needs to know

Which SOURCE addresses should be translated.

It does NOT care about:

- Destination

- TCP

- UDP

- Ports

That's why a Standard ACL is sufficient.

Wildcard

0.0.0.255

Means

192.168.10.x

Packet Flow

Packet arrives

↓

ACL 10

↓

Source = 192.168.10.25

↓

Permit

↓

Translate

Verify

show access-lists

Expected

Standard IP access list 10

permit 192.168.10.0

permit 192.168.30.0

- Step 4 – Create the Public NAT Pool

Requirement

203.0.113.10

↓

203.0.113.15

Command

ip nat pool PUBLIC_NAT 203.0.113.10 203.0.113.15 netmask 255.255.255.248

Breakdown

PUBLIC_NAT

Pool Name

203.0.113.10

First Public IP

203.0.113.15

Last Public IP

255.255.255.248

Pool subnet mask

Why Needed?

Router needs a set of public addresses it can assign during translation.

- Step 5 – Enable PAT

Requirement

Bind ACL 10 to PUBLIC_NAT

Command

ip nat inside source list 10 pool PUBLIC_NAT overload

- Breakdown

ip nat

Configure NAT.

inside

Traffic originates from inside.

source

Translate the source IP.

list 10

Match ACL 10.

Only those networks will be translated.

pool PUBLIC_NAT

Use this public IP pool.

overload

Enable PAT.

Without overload, each internal device would consume one public IP (dynamic NAT).

With overload, many internal devices can share the same public IP(s) by using different source ports.

Packet Flow

192.168.10.10

↓

ACL 10

↓

Match

↓

NAT Pool

↓

203.0.113.10

↓

PAT

↓

Port 1025

↓

Internet

- Verification Commands

show ip nat translations

show ip nat statistics

show access-lists

show ip route

- Troubleshooting Checklist

- Is the default route configured?

show ip route

- Are the correct interfaces marked as ip nat inside and ip nat outside?

- show running-config interface g0/0.10

show running-config interface g0/1

- Does ACL 10 include all internal networks?

show access-lists

- Is the NAT pool configured correctly?

show running-config | include ip nat pool

- Is PAT enabled?

show running-config | include ip nat inside source

- Are translations being created?

show ip nat translations

****………..****

- Objective

Secure the network by controlling traffic entering the IoT VLAN.

- Business Requirement

The company has two VLANs.

Staff VLAN

192.168.10.0/24

IoT VLAN

192.168.30.0/24

The company wants:

✅ IoT devices obtain DHCP addresses

❌ IoT devices cannot ping Internet Server

✅ IoT devices can browse websites

❌ IoT devices cannot access Staff VLAN

✅ Everything else should continue working

Technology Required

- Extended ACL

- Named ACL

- Interface ACL

- Why Extended ACL?

Two ACL types exist.

- Standard ACL

Can filter only

Source IP

Example

access-list 10 permit 192.168.30.0 0.0.0.255

Cannot block HTTP.

Cannot block ICMP.

Cannot match ports.

- Extended ACL

Can filter

- Source

- Destination

- Protocol

- TCP Ports

- UDP Ports

- ICMP

Perfect for firewalls.

- Why Named ACL?

Instead of

access-list 101

Use

EDGE_FIREWALL

Advantages

✔ Easier to read

✔ Easier to edit

✔ Easier to troubleshoot

- Step 1 — Create the ACL

Command

ip access-list extended EDGE_FIREWALL

What does it do?

Creates a named Extended ACL.

Router enters ACL configuration mode.

Prompt changes

Router(config-ext-nacl)#

- Rule A – DHCP Rescue

Requirement

IoT PCs must still receive DHCP addresses.

Why is this needed?

The ACL is applied inbound on the IoT interface.

DHCP Discover packets originate from the IoT clients.

If UDP ports 67 and 68 are blocked, clients will never receive an IP address.

- DHCP Packet Flow

PC

0.0.0.0

↓

DHCP Discover

UDP 68 → UDP 67

↓

Router

↓

DHCP Offer

UDP 67 → UDP 68

↓

PC

- Commands

permit udp any eq bootpc any eq bootps

permit udp any eq bootps any eq bootpc

Equivalent numeric ports:

permit udp any eq 68 any eq 67

permit udp any eq 67 any eq 68

Breakdown

permit

Allow packet.

udp

DHCP uses UDP.

bootpc

Client Port

68

bootps

Server Port

67

- Rule B – Block Ping

Requirement

IoT users must NOT ping

8.8.8.8

Command

deny icmp 192.168.30.0 0.0.0.255 host 8.8.8.8 echo

Breakdown

deny

Drop packet.

icmp

Only ICMP.

HTTP still works.

192.168.30.0

Source Network

host

Exactly one destination.

echo

Only ping requests.

Echo Reply is different.

Packet Flow

PC

↓

Ping

↓

ACL

↓

Rule Match

↓

Dropped

- Rule C – Allow HTTP & HTTPS

Requirement

IoT users should browse websites.

HTTP

permit tcp 192.168.30.0 0.0.0.255 host 8.8.8.8 eq 80

HTTPS

permit tcp 192.168.30.0 0.0.0.255 host 8.8.8.8 eq 443

Why TCP?

Web traffic uses TCP.

HTTP

Port

80

HTTPS

Port

443

Packet Flow

PC

↓

TCP

↓

Port 80

↓

ACL

↓

Permit

↓

Internet

- Rule D – Block Staff Network

Requirement

IoT devices must never access

192.168.10.0/24

Command

deny ip 192.168.30.0 0.0.0.255 192.168.10.0 0.0.0.255

Why

deny ip

Instead of

deny tcp

Because

IP includes

TCP

UDP

ICMP

OSPF

Everything.

Entire network blocked.

Packet Flow

IoT PC

↓

Packet

↓

Destination

192.168.10.x

↓

ACL

↓

Drop

- Rule E – Catch-All

Requirement

Everything else should continue working.

Command

permit ip any any

Why?

Cisco ACLs have an invisible rule.

deny ip any any

This is called

Implicit Deny.

Without the final permit

Everything not previously permitted

↓

Dropped

Packet Flow

Packet

↓

No previous match

↓

permit ip any any

↓

Forward

- Apply ACL

Requirement

Apply inbound on

GigabitEthernet0/0.30

Command

interface GigabitEthernet0/0.30

ip access-group EDGE_FIREWALL in

Why inbound?

Traffic arrives from the IoT PCs.

Instead of letting packets enter the router and then dropping them later, the router discards unwanted traffic immediately.

This is more efficient because it saves CPU and bandwidth.

- Final ACL

ip access-list extended EDGE_FIREWALL

permit udp any eq bootpc any eq bootps

permit udp any eq bootps any eq bootpc

deny icmp 192.168.30.0 0.0.0.255 host 8.8.8.8 echo

permit tcp 192.168.30.0 0.0.0.255 host 8.8.8.8 eq 80

permit tcp 192.168.30.0 0.0.0.255 host 8.8.8.8 eq 443

deny ip 192.168.30.0 0.0.0.255 192.168.10.0 0.0.0.255

permit ip any any

- Verification

show access-lists EDGE_FIREWALL

show ip interface GigabitEthernet0/0.30

show running-config interface GigabitEthernet0/0.30

Verify:

- ACL contains all rules in the correct order.

- EDGE_FIREWALL is applied with ip access-group EDGE_FIREWALL in.

- Troubleshooting Checklist

- Was the ACL applied?

show ip interface GigabitEthernet0/0.30

- Is it applied in the correct direction (in)?

- Are the rules in the correct order?

  - ACLs are processed top to bottom.

  - The first matching rule wins.

- Is permit ip any any at the end?

  - Without it, the implicit deny ip any any blocks all unmatched traffic.

- Are the protocols correct?

  - DHCP → UDP

  - Ping → ICMP

  - HTTP/HTTPS → TCP

- Rule-to-Requirement Mapping

| Requirement | Technology | Command |
| --- | --- | --- |
| IoT clients receive DHCP | Extended ACL | permit udp ... bootpc/bootps |
| Block IoT pings to 8.8.8.8 | Extended ACL | deny icmp ... host 8.8.8.8 echo |
| Allow HTTP | Extended ACL | permit tcp ... eq 80 |
| Allow HTTPS | Extended ACL | permit tcp ... eq 443 |
| Block IoT → Staff | Extended ACL | deny ip 192.168.30.0 0.0.0.255 192.168.10.0 0.0.0.255 |
| Allow all other traffic | Extended ACL | permit ip any any |
| Activate the firewall | Interface configuration | ip access-group EDGE_FIREWALL in |

The two most important concepts to remember are:

- ACL processing is sequential: the router checks rules from top to bottom and stops at the first match.

- Every ACL ends with an invisible deny ip any any, so if you want traffic to continue beyond your specific deny rules, you must explicitly permit it.