- Day 54: Secure Enterprise Perimeter & Internal Routing Integration

- 1. Network Design Philosophy (The "Why")

In this lab, we built a modern, secure branch office network. Instead of putting all the burden on one device, we used Separation of Duties:

- R1-Border (The Security Guard): Sits at the edge of the network connecting to the ISP. Its only job is handling the dangerous outside world. It translates internal IPs to public IPs (NAT) and blocks unauthorized traffic (ACLs).

- R2-Internal (The Office Manager): Sits safely behind R1. Its only job is fast, efficient internal routing (OSPF) between your company's LANs/VLANs.

- SW1 (The Front Desk): Connects the actual end-users (PCs) and enforces physical security (Port Security) so rogue devices can't access the network.

- 2. Phase-by-Phase Configuration Breakdown

Phase 1: Dual-Stack Addressing (IPv4 & IPv6)

Modern networks run IPv4 and IPv6 side-by-side.

- The Concept: You must globally enable IPv6 routing on Cisco routers, or they will act like PCs and refuse to forward IPv6 traffic.

- The Command: ipv6 unicast-routing (Must be done in global config mode).

Phase 2: OSPF and the Default Route

We used OSPFv2 (IPv4) to dynamically share routes inside the company.

- The Problem: R2 has no idea how to reach the Internet. We cannot run OSPF with the ISP, because the ISP doesn't want our routing updates.

- The Solution:

  - Create a static default route on R1 pointing to the ISP: ip route 0.0.0.0 0.0.0.0 203.0.113.2

  - Tell OSPF to "shout" that route down to R2: default-information originate

- The Result: R2 dynamically learns an O*E2 route to the internet without us having to manually configure it.

Phase 3: Dynamic NAT (PAT)

Private IPs (192.168.1.x) cannot be routed on the public internet.

- The Concept: We configure PAT (Port Address Translation / NAT Overload) on R1. R1 will catch packets from the PCs, strip off their private IPs, and slap its own Public IP on them before sending them to the server.

- The Steps:

  - Define the inside/outside boundaries (ip nat inside on Gig0/1, ip nat outside on Gig0/0).

  - Define who is allowed to be translated using an ACL (access-list 1 permit 192.168.1.0 0.0.0.255).

  - Turn on the translation engine (ip nat inside source list 1 interface gig0/0 overload).

Phase 4: Extended ACLs (Traffic Filtering)

We needed to block PC-B from reaching the Internet Server, but allow PC-A.

- The Placement: We placed the ACL on R1's Inside Interface (Gig0/1) Inbound. We have to block the packet before NAT translates it. If we put it on the outside interface, R1 would have already changed PC-B's IP, and the ACL wouldn't recognize it!

- The Commands:

- ip access-list extended BLOCK-PCB

- deny ip host 192.168.1.20 host 203.0.113.2

- permit ip any any   <-- CRITICAL: Without this, the implicit deny blocks everything!

Phase 5: Device Hardening (Port Security & SSH)

We secured the network at the Access Layer and the Management Plane.

- Port Security: Configured on SW1 to physically restrict access. sticky allows the switch to auto-learn PC-A's MAC address and save it to the config. restrict drops hacker traffic and sends a log message without shutting down the whole port.

- SSH: Encrypts our remote management traffic. Requires a hostname, domain name, RSA crypto keys, a local username/password, and enforcing transport input ssh on the VTY lines.

- 3. Verification Commands to Memorize

If the network breaks, these are the commands an enterprise engineer uses to find the problem:

- Routing: show ip route (Look for the O and O*E2 codes).

- OSPF Neighbors: show ip ospf neighbor (Ensure R1 and R2 see each other as 'FULL').

- NAT: show ip nat translations (Look for the private IP mapping to the public IP).

- ACLs: show access-lists (Look for the hit counters to see if packets are being blocked).

- Port Security: show port-security interface fa0/1 (Check the status and violation counters).