Switch is a layer 2 device. The pdu is called frame.

Switch does 3 action:

Learn: switch reads the SOURCE MAC of every incoming frame and records (MAC + port) in its table. It does NOT learn destination MACs by looking them up. It only learns a MAC when THAT device sends a frame.

Flood happens in TWO cases: 1. Destination MAC = broadcast (FF:FF:FF:FF:FF:FF) → always floods

2. Destination MAC = unicast but NOT in MAC table yet → floods until learned

Forward: once the source and destination mac is known the switch only forwards the frame to destination port.

MAC table aging

Default = 300 seconds (5 min)

Exists because devices move/change — keeps table accurate

CLI modes — how to move between them

Switch>            user mode (read-only)

↓ enable

Switch#            privileged mode (full show/clear/save)

↓ configure terminal

Switch(config)#    global config (hostname, passwords)

↓ interface fa0/1

Switch(config-if)# interface config (per-port settings)

To go back: exit (one level) or Ctrl+Z (all the way to #)

Command used today

- Cisco Switch Commands + Description

- enable → Enters privileged EXEC mode (Switch#)

- configure terminal → Enters global configuration mode (Switch(config)#)

- hostname SW1 → Changes switch name to SW1

- enable secret cisco123 → Sets encrypted password for privileged mode

- line console 0 → Enters console port configuration mode

- password cisco → Sets password for console access

- login → Enables password authentication on console

- interface fastethernet 0/1 → Enters configuration for port Fa0/1

- description PC-A → Adds label/description to the interface

- show mac-address-table → Displays MAC address table (learned MACs & ports)

- clear mac-address-table dynamic → Clears dynamically learned MAC addresses

- show running-config → Shows current active configuration (RAM)

- write memory → Saves configuration to startup-config (NVRAM)