# ARP (Address Resolution Protocol)

A networking protocol which is used to map IP address with mac address. 

When ping to another computer the source Doesn’t know the destination mac address, so 1st the ARP runs to find the mac address of destination computer. While ARP the source sets the destination mac address to broadcast FF:FF:FF:FF:FF:FF then the switch broadcasts it to all the pc’s except for the source itself. The device whose ip address matches then replies back to the source with their mac, then only there is connection and actual ping runs (ICMP).

# OSI Layers

Memorization line(From CLaude):

ALL = Application				PDU-> Data

People = Presentation			PDU-> Data

Seem  = Session 				PDU-> Data
To = Transport				PDU-> Segment

Need = Network				PDU-> Packet

Data = Data Link				PDU-> Frame

Processing = Physical			PDU-> Bits

Evidences: