STP (Spanning Tree Protocol) is a Layer 2 protocol that prevents network loops by blocking redundant paths in a switched network. The path reopens if the primary path is not available.

1. Why STP exists

- Ethernet (Layer 2) has no TTL

- Loops cause broadcast storms

- Frames circulate endlessly and multiply traffic

- STP prevents loops by blocking redundant links

2. Root Bridge Election

- Each switch has a Bridge ID = Priority + MAC address

- Lowest Bridge ID becomes Root Bridge

- Default priority = 32768

- If priority is equal → lowest MAC wins

3. How to manually set Root Bridge

To force a switch (example SW3) to become root:

conf t
spanning-tree vlan 1 priority 4096
spanning-tree vlan 10 priority 4096
spanning-tree vlan 20 priority 4096
spanning-tree vlan 99 priority 4096
end

- Lower priority = higher chance to become Root

- 4096 guarantees SW3 wins over default 32768

4. Port Roles

- Root Port (RP): Best path to Root Bridge (one per switch)

- Designated Port (DP): Forwarding port for a segment

- Alternate Port (Altn): Backup path, blocked to prevent loop

5. Port States

- Blocking: No data forwarding, only BPDUs

- Listening: Preparing, no MAC learning

- Learning: Learns MAC table, no forwarding yet

- Forwarding: Normal traffic allowed

- Disabled: Manually shut down

6. Path Cost

- Used to choose best path to Root Bridge

- Lower cost = better path

- Default values:

  - 10 Mbps = 100

  - 100 Mbps = 19

  - 1 Gbps = 4

- Total cost = sum of all link costs to root

7. BPDU (Bridge Protocol Data Unit)

- Sent every ~2 seconds

- Used for:

  - Root election

  - Topology updates

- Even blocked ports send/receive BPDUs

8. Loop Prevention

- Only one active path per segment is allowed

- Redundant link is blocked (Alternate state)

- Prevents broadcast storms

9. Lab Verification Result

- Traffic avoids SW1–SW2 link

- Path goes through SW3 instead

- Blocked link still physically up but not forwarding user traffic