Here is your complete, detailed master reference guide for Layer 2 Security (DHCP Snooping & DAI). It breaks down the exact logic, timing, and commands so you can review it anytime and understand exactly what is happening under the hood.

- Part 1: DHCP Snooping

WHAT is it?

DHCP Snooping is a Layer 2 security feature on a switch that acts like a firewall for DHCP traffic. It filters untrusted DHCP messages and builds a database of legitimate IP addresses assigned to computers on the network.

WHY do we use it?

To stop Rogue DHCP Server Attacks. Without it, a hacker can plug in a laptop, start handing out fake IP addresses, and assign themselves as the Default Gateway. All employee traffic would route through the hacker's laptop (a Man-in-the-Middle attack) before going to the internet.

WHEN do we use it?

You configure this on Access Layer switches (the switches that end-users, PCs, and printers physically plug into). It is the foundational security feature you must turn on before you can use advanced features like DAI or IP Source Guard.

HOW does it work?

It divides all switchports into two categories:

- Trusted Ports: (Usually the uplink to your router/server). The switch allows DHCP OFFERS (IP address assignments) to flow through this port.

- Untrusted Ports: (Where the PCs plug in). The switch blocks any DHCP OFFERS coming from these ports. If a hacker tries to act like a server, the switch kills their packets.

- The Database: Every time a legitimate PC gets an IP address from the real server, the switch secretly writes down the PC's MAC address, IP address, and port number into a file called the DHCP Snooping Binding Table (The VIP List).

- Part 2: Dynamic ARP Inspection (DAI)

WHAT is it?

DAI is a Layer 2 security feature that validates ARP packets (the messages computers use to match an IP address to a physical MAC address).

WHY do we use it?

To stop ARP Poisoning (ARP Spoofing). Hackers send fake ARP messages to trick PCs into thinking the hacker's MAC address belongs to the Router. DAI prevents computers from being lied to.

WHEN do we use it?

Immediately after DHCP Snooping is configured and has had time to build its database. (DAI is completely blind without the DHCP database).

HOW does it work?

- Like DHCP Snooping, DAI uses Trusted (router) and Untrusted (employee) ports.

- The Bouncer Logic: When an ARP packet arrives on an Untrusted port, DAI intercepts it. DAI looks at the DHCP Snooping Binding Table.

- If the PC's MAC and IP match the list perfectly, the packet is forwarded.

- If the PC is using a static IP, or spoofing someone else's IP, they are not on the list, and DAI instantly destroys the packet.

The Static IP Exception: Because servers and printers use static IPs, they are not in the DHCP database. To prevent DAI from dropping their traffic, engineers must manually write an ARP ACL to give them a permanent bypass pass.

- Part 3: The Command Glossary

Here is every command you used today, and exactly what it does behind the scenes:

⚙️ Configuration Commands (Global & Interface)

ip dhcp snooping

- What it does: Turns on the master DHCP Snooping engine globally on the switch.

- Why: Without this, the switch is completely deaf to the security rules.

ip dhcp snooping vlan 1

- What it does: Activates the snooping engine specifically for VLAN 1.

- Why: You have to specify which VLANs to protect, otherwise the engine runs but guards nothing.

ip dhcp snooping trust

- What it does: Applied to an interface (like Fa0/1), this tells the switch: "The legitimate DHCP Server lives down this wire."

- Why: This allows DHCP OFFERS to pass through this specific port.

no ip dhcp snooping information option

- What it does: Stops the switch from injecting Option 82 tracking data (the "Post-It Note") into the client's DHCP request.

- Why: Standard Cisco routers will drop DHCP packets that have been altered. This command forces the switch to leave the packet in its original state so the router will accept it.

ip arp inspection vlan 1

- What it does: Turns on the DAI engine and applies it to VLAN 1.

- Why: Tells the switch to start intercepting and investigating every single ARP packet on the VLAN.

ip arp inspection trust

- What it does: Applied to an interface (like Fa0/1), this tells DAI to stop inspecting ARP packets on this specific port.

- Why: We trust the router. If we inspect the router and the router isn't in our DHCP database, we would accidentally block the router and break the whole network!

🔍 Verification & Troubleshooting Commands

show ip dhcp snooping

- What it does: Displays the foundational configuration.

- When to use it: Use this to verify that snooping is turned on, check which VLANs are protected, verify Option 82 is disabled, and see which ports are trusted vs untrusted.

show ip dhcp snooping binding

- What it does: Prints the "VIP Guest List."

- When to use it: Use this to see exactly which PCs successfully pulled a DHCP address. If a PC is not on this list, it cannot send ARP packets on the network.

show ip arp inspection interfaces

- What it does: Displays the Bouncer assignments.

- When to use it: Use this to verify that your router uplink is set to "Trusted" and all your employee PC ports are set to "Untrusted."

show ip arp inspection statistics

- What it does: Shows the hit counters for packets that were forwarded and dropped by DAI.

- When to use it: Use this to see if the switch is actively catching a hacker (or dropping a legitimate static IP). Note: As discovered, Packet Tracer's bug causes these counters to remain at 0 even when they shouldn't.

💻 PC/End-User Commands

arp -d

- What it does: Run from the Windows/PC Command Prompt. It deletes the PC's internal ARP memory (the ARP cache).

- When to use it: Use this when you need to force a PC to send a brand new, fresh ARP Request to test if the switch's DAI filter is actually working.