- Multi-Building Enterprise Network (VLAN, STP & EtherChannel)

- 🧩 1. Objective

To design and implement a multi-building enterprise switched network using VLAN segmentation, STP for loop prevention, VTP for VLAN distribution, and EtherChannel for link aggregation using Cisco Packet Tracer.

- 🏢 2. Network Topology Overview

Company: ABC Corp

- 2 Buildings

- 3 Switches (SW-CORE, SW-A, SW-B)

- Multiple departments connected via VLANs

- 🌐 3. VLAN & IP Design

VLANs

- VLAN 10 → HR

- VLAN 20 → Finance

- VLAN 30 → IT

- VLAN 99 → Management (Native VLAN)

IP Network

- Base: 192.168.0.0/16

- HR → 192.168.10.0/24

- Finance → 192.168.20.0/24

- IT → 192.168.30.0/24

- 🔧 4. Device Roles

SW-CORE

- VTP Server

- Root Bridge (STP priority 4096)

- VLAN creation switch

SW-A / SW-B

- VTP Clients

- Access + trunk switches

- EtherChannel participants

- ⚙️ 5. Full Configuration Commands

- 🔹 SW-CORE Configuration

enable
configure terminal

hostname SW-CORE

vtp domain ABC-CORP
vtp mode server

vlan 10
 name HR
vlan 20
 name Finance
vlan 30
 name IT
vlan 99
 name Management

spanning-tree mode rapid-pvst
spanning-tree vlan 1,10,20,30,99 priority 4096

Trunk Configuration (SW-CORE)

interface fa0/1
 switchport mode trunk
 switchport trunk native vlan 99

interface fa0/2
 switchport mode trunk
 switchport trunk native vlan 99

- 🔹 SW-A Configuration

enable
configure terminal

hostname SW-A

vtp domain ABC-CORP
vtp mode client

spanning-tree mode rapid-pvst

Trunk + Access Ports

interface fa0/1
 switchport mode trunk
 switchport trunk native vlan 99

interface fa0/2
 switchport mode access
 switchport access vlan 10
 spanning-tree portfast
 spanning-tree bpduguard enable

interface fa0/3
 switchport mode access
 switchport access vlan 20
 spanning-tree portfast
 spanning-tree bpduguard enable

EtherChannel (SW-A side)

interface range fa0/4 - 5
 channel-group 1 mode active

interface port-channel 1
 switchport mode trunk

- 🔹 SW-B Configuration

enable
configure terminal

hostname SW-B

vtp domain ABC-CORP
vtp mode client

spanning-tree mode rapid-pvst

Trunk + Access Ports

interface fa0/1
 switchport mode trunk
 switchport trunk native vlan 99

interface fa0/2
 switchport mode access
 switchport access vlan 20
 spanning-tree portfast
 spanning-tree bpduguard enable

interface fa0/3
 switchport mode access
 switchport access vlan 30
 spanning-tree portfast
 spanning-tree bpduguard enable

EtherChannel (SW-B side)

interface range fa0/4 - 5
 channel-group 1 mode active

interface port-channel 1
 switchport mode trunk

- 🔗 6. Verification Commands

show vlan brief
show interfaces trunk
show vtp status
show spanning-tree
show etherchannel summary

- 📊 7. Expected Outcome

- VLAN segmentation works correctly

- PCs communicate within same VLAN

- STP prevents loops using Rapid-PVST+

- SW-CORE acts as root bridge

- EtherChannel bundles links successfully

- Trunk links carry VLANs 10, 20, 30, 99

- 🏁 8. Conclusion

This project demonstrates a complete Layer 2 enterprise network design using VLAN segmentation, VTP-based VLAN distribution, STP loop prevention, and EtherChannel link aggregation. It simulates real-world campus network architecture and builds foundational CCNA-level switching skills.