- 📓 CCNA Master Study Notes: Day 45 — IPv6 Router Configuration & Static Routing (Updated)

- 🔄 Day 44 vs. Day 45: What’s New Today?

While Day 44 focused on basic IPv6 host addressing (SLAAC, EUI-64, and Link-Local generation), Day 45 builds directly on top of it by introducing multi-hop network forwarding across a core infrastructure.

| Feature / Topic | 🗓️ Day 44 (IPv6 Addressing) | 🗓️ Day 45 (IPv6 Router Configuration) |
| --- | --- | --- |
| Primary Focus | Host IP generation & local Link-Local connectivity | Multi-hop routing tables & static route variations |
| Topology Scale | 2-Router simple edge link (R1↔R2) | 3-Router enterprise chain (R1↔R2↔R3 + Loopbacks) |
| Static Route Types | Basic Next-Hop GUA | Next-Hop GUA, Default Routes (::/0), and Fully Specified Routes |
| Virtual Interfaces | Physical ports only | Loopback interfaces (Loopback0) simulating internal core subnets |

- 📑 Section 1: Core IPv6 Mechanics (Why, What, When, How)

1. ipv6 unicast-routing

- WHY: Cisco routers ship with IPv6 forwarding disabled by default. Without this command, the router acts only as a silent IPv6 host and drops transit traffic.

- WHAT: Wakes up the IPv6 routing engine, builds the IPv6 routing table, and triggers the router to send periodic Router Advertisements (RAs).

- WHEN: Must be executed first in global configuration mode on every IPv6 router.

- HOW:

Plaintext

Router(config)# ipv6 unicast-routing

2. Router Advertisements (RA) & SLAAC

- WHY: Allows hundreds of end-user computers to connect to the network without typing 128-bit addresses or managing complex DHCP servers.

- WHAT:

  - Router Solicitation (RS - ICMPv6 Type 133): Sent by a booting host asking, "Is there a router here?"

  - Router Advertisement (RA - ICMPv6 Type 134): Sent by the router telling the host the network prefix (/64) and default gateway (FE80::x).

- WHEN: Used for high-density end-user client subnets (phones, desktops, laptops).

- HOW: Enabled automatically as soon as ipv6 unicast-routing is turned on and an IPv6 GUA address is added to a router interface.

3. Global Unicast (GUA) vs. Link-Local (LLA)

- Global Unicast Address (GUA):

  - WHAT: Publicly routable IPv6 address (starts with 2001:, 2002:, or 3...).

  - WHY: Allows packets to traverse multiple routers across the internet.

  - HOW: ipv6 address 2001:DB8:1::1/64

- Link-Local Address (LLA):

  - WHAT: Non-routable management address starting with FE80::/10.

  - WHY: Used for neighbor discovery (NDP), local communication, and explicit next-hop routing gateways. Routers never forward LLA traffic past the local link.

  - HOW: ipv6 address FE80::1 link-local

- 🛠️ Section 2: The 4 Static Route Syntax Types

Plaintext

R(config)# ipv6 route <destination-prefix/prefix-length> {next-hop-ipv6 | exit-interface} [next-hop-link-local]

| Route Type | Example Command Syntax | Exam & Real-World Use Case |
| --- | --- | --- |
| Next-Hop GUA (Recursive) | ipv6 route 2001:db8:3::/64 2001:db8:12::2 | Standard static route pointing directly to the next router's public IPv6 address. |
| Directly Connected | ipv6 route 2001:db8:3::/64 GigabitEthernet0/0 | Points straight out a point-to-point exit interface. |
| Fully Specified (Link-Local) | ipv6 route 2001:db8:3::/64 GigabitEthernet0/0 FE80::2 | Crucial CCNA Topic: Because Link-Local addresses (FE80::2) exist on every link, you must specify the local exit interface alongside the LLA. |
| IPv6 Default Static Route | ipv6 route ::/0 2001:db8:12::2 | Matches all unknown destination networks using ::/0 (the IPv6 equivalent of IPv4's 0.0.0.0 0.0.0.0). |

- 🗺️ Section 3: Topology & Addressing Blueprint

Plaintext

[ PC-1 ]                                                                                           [ PC-2 ]

(Branch A)                                                                                         (Branch B)

|                                                                                                  |

(G0/1)                                                                                             (G0/1)

[ R1 ] (G0/0) <======== 2001:db8:12::/64 ========> (G0/0) [ R2 ] (G0/1) <======== 2001:db8:23::/64 ========> (G0/0) [ R3 ]

(Branch A)                WAN Link A-to-Core              (Core/HQ)               WAN Link Core-to-B             (Branch B)

G0/1: 2001:db8:1::1/64                                 Lo0: 2001:db8:22::1/64                          G0/1: 2001:db8:3::1/64

LLA: fe80::1                                           LLA: fe80::2                                    LLA: fe80::3

- Branch A LAN (R1 G0/1): 2001:DB8:1::/64 | R1: 2001:DB8:1::1 | LLA: FE80::1

- WAN Link 1 (R1 G0/0 <-> R2 G0/0): 2001:DB8:12::/64 | R1: 12::1 | R2: 12::2

- Core Management (R2 Loopback0): 2001:DB8:22::1/64

- WAN Link 2 (R2 G0/1 <-> R3 G0/0): 2001:DB8:23::/64 | R2: 23::2 | R3: 23::3

- Branch B LAN (R3 G0/1): 2001:DB8:3::/64 | R3: 2001:DB8:3::1 | LLA: FE80::3

- 📜 Section 4: Complete CLI Configuration Scripts

🏢 Router 1 (Branch A Edge)

Plaintext

enable

configure terminal

hostname R1

! --- Enable the IPv6 Engine ---

ipv6 unicast-routing

! --- Branch A LAN Interface ---

interface GigabitEthernet0/1

description Branch A LAN

ipv6 address 2001:DB8:1::1/64

ipv6 address FE80::1 link-local

no shutdown

exit

! --- WAN Interface to R2 ---

interface GigabitEthernet0/0

description WAN to R2 Core

ipv6 address 2001:DB8:12::1/64

ipv6 address FE80::1 link-local

no shutdown

exit

! --- Default Static Route (Send all unknown traffic to Core) ---

ipv6 route ::/0 2001:DB8:12::2

end

write memory

🏢 Router 2 (Core Router)

Plaintext

enable

configure terminal

hostname R2

! --- Enable the IPv6 Engine ---

ipv6 unicast-routing

! --- Core Loopback Interface ---

interface Loopback0

description Core Virtual Subnet

ipv6 address 2001:DB8:22::1/64

exit

! --- WAN Interface to R1 ---

interface GigabitEthernet0/0

description WAN to R1

ipv6 address 2001:DB8:12::2/64

ipv6 address FE80::2 link-local

no shutdown

exit

! --- WAN Interface to R3 ---

interface GigabitEthernet0/1

description WAN to R3

ipv6 address 2001:DB8:23::2/64

ipv6 address FE80::2 link-local

no shutdown

exit

! --- Static Route to Branch A LAN ---

ipv6 route 2001:DB8:1::/64 2001:DB8:12::1

! --- Static Route to Branch B LAN ---

ipv6 route 2001:DB8:3::/64 2001:DB8:23::3

end

write memory

🏢 Router 3 (Branch B Edge)

Plaintext

enable

configure terminal

hostname R3

! --- Enable the IPv6 Engine ---

ipv6 unicast-routing

! --- WAN Interface to R2 ---

interface GigabitEthernet0/0

description WAN to R2 Core

ipv6 address 2001:DB8:23::3/64

ipv6 address FE80::3 link-local

no shutdown

exit

! --- Branch B LAN Interface ---

interface GigabitEthernet0/1

description Branch B LAN

ipv6 address 2001:DB8:3::1/64

ipv6 address FE80::3 link-local

no shutdown

exit

! --- Fully Specified Default Route (Using LLA + Exit Interface) ---

ipv6 route ::/0 GigabitEthernet0/0 FE80::2

end

write memory

- 🖥️ Section 5: PC Configurations & Packet Tracer Quirks

1. SLAAC (Automatic) vs. Static PC Setup

- SLAAC Method (Default in Lab):

  - Set IPv6 configuration to Automatic.

  - PC receives prefix 2001:DB8:3::/64 from R3 and builds a long EUI-64 host ID (e.g., 2001:DB8:3:0:20C:CFFF:FE6B:D063).

  - Gateway automatically fills in as FE80::3.

- Static Method (Manual):

  - Set IPv6 configuration to Static.

  - IPv6 Address: 2001:DB8:3::10

  - Subnet Prefix Length: 64

  - IPv6 Default Gateway: 2001:DB8:3::1

2. Critical Packet Tracer Troubleshooting Rules

⚠️ Packet Tracer Rule #1 (The Slash Bug): Never type /64 inside the IPv6 address field in Packet Tracer! Doing so erases the address when you switch windows. Put 2001:DB8:3::10 in the top box and 64 in the Subnet Prefix Length box below it.

⚠️ Packet Tracer Rule #2 (SLAAC Ping Target Mismatch): If a PC is set to Automatic, pinging 2001:DB8:3::10 will time out because the PC generated a dynamic EUI-64 address instead. Either ping the exact EUI-64 string from ipconfig or switch the PC to Static.

- 🔬 Section 6: Diagnostic & Verification Toolkit

1. View Operational Interfaces

Plaintext

R1# show ipv6 interface brief

- Verification: Confirms up/up line status, active GUAs, and assigned Link-Local addresses.

2. Audit the Routing Table

Plaintext

R1# show ipv6 route

- S ::/0: Active static default route.

- C: Directly connected network.

- L: Local host route (/128) pointing to the router's own interface address.

3. Verification Pings

- From PC-1 Command Prompt to PC-2 (End-to-End):

DOS

ping 2001:DB8:3::10

- From Router CLI to Link-Local Address:

Plaintext

R1# ping fe80::2

Output interface: GigabitEthernet0/0

- Extended Source Ping (Test return path from LAN interface):

Plaintext

R1# ping 2001:DB8:3::10 source gigabitEthernet 0/1