For subnetting on demand, you will need to know how many hosts re required in each department assuming there are many departments,

You start with that department having highest number of hosts (End device)

Step 1= find number of hosts required

Step 2= Add +2 (for network and broadcast ip)

Step 3= find the smallest power of 2 which is >= number,  2^x >= Number

The X is host bit,

Find prefix, 32-X , this prefix gives us Subnet mask

Eg: if prefix was 26, then it would mean 26 “1”, 11111111.11111111.11111111.11000000 -> 255.255.255.192 (Subnet mask)

Step 4= find the block size, which is 2^x

Eg 2^6=64

Start = 0

Block = 64 addresses

End = Start + Block - 1

End = 0 + 64 - 1

End = 63

Then the ip ranges from 0-63, (Starting from 0 so upto 63)

Now usable ip ranges from 1-62, 0 for network and 63 forbroadcast

Similarly for another department

Do same steps buts keep in mind that next ip range starts from previous department range 64-… ,  here also 1st is used for network and last is used for broadcast.

Vlsm(Variable length subnet mask)= it is used to minimize the wasted subnetmasks.

different prefix lengths for different subnets instead of giving everyone the same size /24 each subnet gets exactly the size it needs.

Eg: Without VLSM (/24 for all) → 924 addresses wasted

With VLSM                  → 20  addresses wasted

Subnetting helps to keep different isolated saving resources and time also with improving security.