Network Address Translation (NAT) is the ultimate bouncer of the internet. It takes private, "illegal" LAN IP addresses (192.168.33.X) and swaps them for a single public, "legal" WAN IP address so your traffic can survive on the public internet. If one tiny setting is off, the internet boundary breaks.

🛠️ The 3 Golden Troubleshooting Commands

- show ip nat translations

  - What it does: Displays the active connection logbook inside the router.

  - The Blueprint: Shows the Inside Local (the PC's real private IP) mapped perfectly to the Inside Global (the public IP the router uses to mask the PC). If this table is completely empty while you are actively pinging, NAT is failing.

- show ip nat statistics

  - What it does: Gives a high-level health report of the NAT engine.

  - The Blueprint: Instantly tells you which ports are designated as Inside and Outside. If either category is blank, the router is blind and will refuse to translate anything. It also tracks "Hits" (successful translations) and "Misses" (failed translations).

- debug ip nat

  - What it does: Turns on live, real-time tracking on your console screen.

  - The Blueprint: Every single time a packet hits the router, you see the exact swap happen live (e.g., s=192.168.33.10 -> 203.0.113.1). If you run a ping and this tool stays completely silent, the traffic isn't even reaching the router's NAT engine.

🚨 The "Hall of Shame" — 4 Common NAT Traps

- The Blind Border (Missing Inside/Outside Command)

  - The Issue: Forgetting to type ip nat inside on the LAN interface or ip nat outside on the WAN interface.

  - The Result: The router forwards the raw private IP (192.168.33.X) out to the internet. The ISP sees a private IP on a public wire and instantly deletes the packet.

- The Mismatched Access-List (ACL)

  - The Issue: Defining a PAT rule for Access-List 1, but writing an ACL that doesn't match your actual subnet (e.g., allowing 192.168.10.0 when your PCs are actually using 192.168.33.0).

  - The Result: The router doesn't recognize the traffic as something it's allowed to translate, so it ignores it.

- The Missing Gateway Exit

  - The Issue: Forgetting to assign the Default Gateway on the local end-devices or switches.

  - The Result: The PCs can talk to each other across the local switch, but they don't know where the "exit door" to the router is, meaning the traffic never leaves the local LAN.

- Pool Exhaustion (Dynamic NAT Only)

  - The Issue: Having a pool of 5 public IP addresses but 50 computers trying to access the internet at the same time without using the overload keyword.

  - The Result: The first 5 computers get online; the remaining 45 get completely blocked until someone logs off.