# Binary, Hex & IP Addresses

Bit table

128 64 32 16 8 4 2 1

ipV4 = 4 octets  192.168.1.10

1 Octet = 8 bits= max value 255

/24 in subnet masks means there are 24 “1” out of total 32 so remaining 8 are “0”. In these 8 “0” it means that the value of host portion  can be from 0 to 255.

However the 0 is perused for network address and 255 is used for broadcast , so usable are only from 1 to 254.

To find out ip address and mac address in windows use command: ipconfig /all

To find out mac table of switch in its CLI type show mac-address-table

Mac address is written in hex value . it consists of 6 groups with each group having 2 hex values.  1 hex = 4 bits, so each group is 8 bits.

Decimal to binary conversion:

Suppose decimal is 190

Then, 1st step will be writing the bit values 128 , 64,32,16,8,4,2,1

Then take the number and analyse it if its greater or equal to 1st element of bit table i.e 128, if its true then write 1 or else false. 192>=128 so 1st bit value 1

Next subtract the bit value’s from original number only if it was true.

i.e 190-128=62

now the 2nd number analyse with 2nd bit table element, 62>= 64 false, so 2nd bit value is 0

if false so not subtracting

same number is used for bit 3 , then

62>=32 , true, 3rd  bit value is 1, again subtract and do same when true ,

And write in sequence , 10111110

Hex values have 0 1 2 3 4  5 6  7 8 9 A B C D E F

EVIDENCES: